# Test Items 详细说明

## Item 1 - Requirement 1.0: Arbiter 1 fixed priority check

Requirement ID: 1.0

Feature: Cache must arbitrate correctly between multiple clients.

Sub Feature: The cache must ensure requesters are served based on their priority (strict priority).

Feature Description: Cache must arbitrate correctly between multiple clients.

Test Case: Arbiter 1 fixed priority check

Verification Goal: Purpose:  Arbiter shall always prioritise a high priority requester. 
Method: A SV based model is used to do arbiter verification. The model is bind directly to arbiter instance in HPDcache RTL.
Link: hpdcache_fxarb_sva.sv

Coverage Method: Assertion Coverage

## Item 2: Ready is one hot  (Abiter 1)

Sub Feature: The cache must never simultaneously grant two requesters.

Test Case: Ready is one hot  (Abiter 1)

Verification Goal: Purpose: Arbiter shall except only one request at a time.
Method: A SV based model is used to do arbiter verification. The model is bind directly to arbiter instance in HPDcache RTL.
Link: hpdcache_fxarb_sva.sv

Coverage Method: Assertion Coverage

## Item 3: Shcmoo 5 (hpdcache_req  (@NREQ= X) -> hpdcache_req (@NREQ=Y) )

Test Case: Shcmoo 5 (hpdcache_req  (@NREQ= X) -> hpdcache_req (@NREQ=Y) )

Verification Goal: Purpose: This schmoo is intended to cover the temporal relationship between 2 processor issuing a request at the same time.  If pys_indexed = 0, cache needs 2 cycle to read the address, 1 st cycle it (arbiter) accepts the request and second cycle is used to accept the virtual addresse. 
If another processor makes a request at this second cycle, we have 2 requests coming at the same time. Cover the case where NOC ready always zero, one or random during the duration of assertion
Description:
hpdcache_req_X_valid (ld/st/cmos/amos) (p_i = 0/1) -> #N hpdcache_req_Y_valid (ld/st/cmos/amos)
N = [-5 : 5] 
p_i = phys_indexed 

Trigger and clocking events: The clock is core clock. Assertion is trigerred by the request 1 valid. 
Number of Cases: 11x2x4x4x2 =704

Coverage Method: Assertion Coverage

## Item 4: Arbiter 1 work conserving check

Sub Feature: If neither of the following 5 conditions are true, and requestors are presenting requests, then a request must be accepted (1=RTAB not full, 2=no CMO fence in progress, 3=no uncacheable or atomic operation is in progress, 4=no load miss in stage 1 of the pipeline, 5=there is no load in stage 1 or the current request is not a store).

Test Case: Arbiter 1 work conserving check

Verification Goal: Purpose: Verify  if arbiter is ready, it shall always serve a request. 
Method: A SV based model is used to do arbiter verification. The model is bind directly to arbiter instance in HPDcache RTL.
Link: hpdcache_fxarb_sva.sv

Coverage Method: Assertion Coverage

## Item 5 - Requirement 2.0: test_hpdcache_multiple_load_store_requests

Requirement ID: 2.0

Feature: Requester interface must suport read + write operations.

Sub Feature: The HPDCACHE_REQ_LOAD operation must trigger a read operation on the cache.

Feature Description: Requester interface must suport read + write operations.

Test Case: test_hpdcache_multiple_load_store_requests

Verification Goal: This test is derived from test 1.1.1. 
Stimuli:   In this test 80% of the traffic is load and store.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 6: test_hpdcache_multiple_load_store_requests_in_region

Test Case: test_hpdcache_multiple_load_store_requests_in_region

Verification Goal: Same as 2.1.4. 
Stimuli: This test performs  80% load store accesses within randomized address regions and the other accesses are fully random but within the regions.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 7: Back Pressure with load at the same address (offset random)

Test Case: Back Pressure with load at the same address (offset random)

Verification Goal: Purpose :  The purpose of this test is to verify the number of requests accepted by the MSHR and rtab in the case where acceses are made at the consecutive SETS.  The number of accepted requests depends on rtab_single_flag. 
Target:  MSHR and RTAB mecanism 
Scenerio: In this test, the following sequence  is run with memory reponses disabled for first 1000 cycles. We then check the number of requests accepted by the cache, just before the memory reponses are enabled

    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end

Stimuli: 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :  In this test OP is fixed to HPDCACHE_REQ_LOAD. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets  are random.

The maximum number of requests will be actually dependant of min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS, HPDCACHE_SETS). Here you should replace the 8 by RTAB_ENTRIES

Criteria Pass Fail: Criteria (PASS/FAIL) : 
We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  
Expected number of request the moment back pressure is released after 1000 cycles

get_req_counter() == (m_cfg_rtab_single_entry == 1) ? HPDCACHE_SETS + 1: HPDCACHE_SETS +RTAB_ENTRIES

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 8: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of MSHR, RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be NUM_WBUF_ENTRIES.  If the value does not match the expected a UVM_ERROR is flaged. 
Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 9: Back Pressure with load at the same address (offset random)

Test Case: Back Pressure with load at the same address (offset random)

Verification Goal: Purpose :  The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where acceses are made at the consecutive SETS.  The number of accepted requests depends on rtab_single_flag. 
Target:  MSHR and RTAB mecanism 
Scenerio: In this test, the following sequence  is run with memory reponses disabled for first 1000 cycles. We then check the number of requests accepted by the cache, just before the memory reponses are enabled

    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end

Stimuli: 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :  In this test OP is fixed to HPDCACHE_REQ_LOAD. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets  are random.

The maximum number of requests will be actually dependant of min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS, HPDCACHE_SETS). Here you should replace the 8 by RTAB_ENTRIES

Criteria Pass Fail: Criteria (PASS/FAIL) : 
We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  
Expected number of request the moment back pressure is released after 1000 cycles

get_req_counter() == (m_cfg_rtab_single_entry == 1) ? HPDCACHE_SETS + 1: HPDCACHE_SETS +RTAB_ENTRIES

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 10: Read only with large memory back pressure

Test Case: Read only with large memory back pressure

Verification Goal: Purpose: The aim of this performance test is to show the benefits of having a MSHR on the performance. 
Check the bandwidth of cached reads at the core interface of HPDcache when a read arrives with no inter request delay. 
If memory response reply with a delay of 2*MSHR_SETS*MSHR_WAYS cycle(more realistic delay), the cache should be able to accept a read miss every third cycle. 
Tarrget:    Load Performance maximising the use of MSHR 
Scenario: Load only test
Stimulie: Test uses random access sequence. 
Requirements: 
Following condition are respected to get full performance 
1. Load accesses are cacheable 
2. There is no conflit within the MSHR (An access should not be made on a SET which already has an ongoing miss) 
3. There is no inter request delay @core interface 
4. Memory response model reply within minimum delay possible 
Constraints: 
The wbuf and rtab should be configurered to get the maximum outuput. Following values are used:
                 m_cfg_rtab_single_entry = 0

Criteria Pass Fail: Criteria (PASS/FAIL): 
A check is performed at the end of test in report phase. 
The cache is expected to accept  NB_TXN loads in 2*3*NB_TXN cycles.
I would generate the address as above (cf 2.5.2)

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 11: Mem Access Check

Test Case: Mem Access Check

Verification Goal: Purpose: Check to see if hpdcache makes a correct type of memory request on NoC interface. 
Method: This check is performed when a new memory request is observed on the NoC interface

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 12: OP coverage (meaningfull)

Test Case: OP coverage (meaningfull)

Verification Goal: Purpose: Coverage that every (following) operation is followed by every operation on the same address followed by a load on the same addresse.  
STORE
AMO_LR
AMO_SC
AMO_SWAP
AMO_ADD
AMO_AND
AMO_OR
AMO_XOR
AMO_MAX
AMO_MAXU
AMO_MIN
AMO_MINU
CMO
Sampling Event: The coverage is sampled when a valid request is recieved. 
Buckets: Bucket : 13x13 == 169

Coverage Method: Functional Coverage

## Item 13: test_hpdcache_multiple_load_store_requests

Sub Feature: The HPDCACHE_REQ_STORE operation must trigger a write operation on the cache.

Test Case: test_hpdcache_multiple_load_store_requests

Verification Goal: This test is derived from test 1.1.1. 
Stimuli:   In this test 80% of the traffic is load and store.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 14: test_hpdcache_multiple_load_store_requests_in_region

Test Case: test_hpdcache_multiple_load_store_requests_in_region

Verification Goal: Same as 2.1.4. 
Stimuli: This test performs  80% load store accesses within randomized address regions and the other accesses are fully random but within the regions.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 15: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? HPDCACHE_WBUF_DIR_ENTRIES: 1) +  num_rtab_entry;
        else 
            num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
        end 
        default: 
        begin
          num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES; 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 16: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? HPDCACHE_WBUF_DIR_ENTRIES : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
              default :    num_expected_write =CNT*HPDCACHE_WBUF_DIR_ENTRIES; 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 17: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of MSHR, RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be NUM_WBUF_ENTRIES.  If the value does not match the expected a UVM_ERROR is flaged. 
Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 18: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES): 1) +  num_rtab_entry;
        else 
            num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
        end 
        default: 
        begin
          num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 19: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
              default :    num_expected_write =CNT*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 20: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES).  If the value does not match the expected a UVM_ERROR is flaged.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 21: Write only with limited back pressure

Test Case: Write only with limited back pressure

Verification Goal: Purpose :  The aim of this performance test is to show the benefits of having a write buffer on the performance. 
Check the bandwidth of cached write at the core interface of HPDcache when writes arrive with no inter request delay. 
If memory response reply within a delay < NUM_WBUF cycles, the cache should be able to accept every write. 
Target     : Write Performance 
Scenario:  Write only test. 
Stimuli   :  Test is derived from random access tests.  
Requirement : 
Following condition should be respected to get full performance 
1. Store accesses are cacheable 
2. There is not inter request delay @core interface 
3. Memory response model reply within minimum (< NUM_WBUF cycles) delay possible 

Constraints: 
OP = HPDCACHE_REQ_STORE, uncacheable=0. 
The wbuf and rtab should be configured to get the maximum output. Following values are used:
                 m_cfg_wbuf_threshold = > 2 < HPDCACHE_WBUF_DIR_ENTRIES
                 m_cfg_wbuf_reset_timecnt_on_write = 1;
                 m_cfg_wbuf_sequential_waw = 0;
                 m_cfg_wbuf_inhibit_write_coalescing = 0;
                 m_cfg_rtab_single_entry = 0

All writes are done at unique SET where no request is on the fly.

Criteria Pass Fail: 
Criteria (PASS/FAIL): 
A check is performed at the end of test in report phase.
The cache is expected to accept  NB_TXN stores in NB_TXN cycles. 

In the case where write_buf_dir_entry, write_buf_data_entry < the memory write latency,  almost 3 times degrdation of performance is observed.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 22: OP coverage (meaningfull)

Test Case: OP coverage (meaningfull)

Verification Goal: Purpose: Coverage that every (following) operation is followed by every operation on the same address followed by a load on the same addresse.  
STORE
AMO_LR
AMO_SC
AMO_SWAP
AMO_ADD
AMO_AND
AMO_OR
AMO_XOR
AMO_MAX
AMO_MAXU
AMO_MIN
AMO_MINU
CMO
Sampling Event: The coverage is sampled when a valid request is recieved. 
Buckets: Bucket : 13x13 == 169

Coverage Method: Functional Coverage

## Item 23 - Requirement 3.0: Byte enable is aligned with offset

Requirement ID: 3.0

Feature: Requesters must provide valid requests.

Sub Feature: The LSBits of the address (LOAD/STORE) must be aligned to the SIZE (naturally aligned).

Feature Description: Requesters must provide valid requests.

Test Case: Byte enable is aligned with offset

Verification Goal: Check if byte enable is aligned with req offset

Criteria Pass Fail: prop_core_req_be_align

## Item 24: Byte enable coverage

Test Case: Byte enable coverage

Verification Goal: Purpose: The purpose is to see the impact of byte enable disabled or enabled. 
Coverage that each bit of the be signal was enable/disable once, and that the extreme value ( only '0 and only '1) are covered with 1 bucket each.
Cover in the case of responses with or without error. 
Sampling Event: Once a response is recieved 
Buckets: HPDCACHE_REQ_DATA_WIDTH/2*2 + 2(extreme cases)

Coverage Method: Functional Coverage

## Item 25: Size coverage

Sub Feature: The BE value from the requester must be naturally aligned to the request size (bits outside the range [2^SIZE…2^(SIZE+1)-1] must be zero. This implies the total number of 1 bits must be <= 2^SIZE.

Test Case: Size coverage

Verification Goal: Purpose: Coverage of each value of this signal 
Buckets: 3 bits => 8 buckets

Coverage Method: Functional Coverage

## Item 26: Cross coverage between the byte enable and the size

Test Case: Cross coverage between the byte enable and the size

Verification Goal: Purpose: Cover that for each size, every byte enable is used. 
Sampling Event: The coverage is sampled when a valid request is recieved. 
Bucket: ??

Coverage Method: Functional Coverage

## Item 27: Cross coverage between the size, the word and op

Test Case: Cross coverage between the size, the word and op

Verification Goal: Purpose: Cover that for each size, every word and opcode. 
Sampling Event: The coverage is sampled when a valid request is recieved. 
Bucket: ??

Coverage Method: Functional Coverage

## Item 28: Byte enable is aligned with size

Test Case: Byte enable is aligned with size

Verification Goal: Check if byte enable is aligned with requester size

## Item 29 - Requirement 4.0: Mem Addr Check

Requirement ID: 4.0

Feature: Cache must relay requests correctly from requestors to memory interface and vice versa

Sub Feature: The request address must be correctly transferred from requestors to the memory write interface.

Feature Description: Cache must relay requests correctly from requestors to memory interface and vice versa

Test Case: Mem Addr Check

Verification Goal: Purpose: Check to see if hpdcache makes an access at correct address on NoC interface. 
Method: This check is performed when a new memory request is observed on the NoC interface

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 30: Offset coverage

Test Case: Offset coverage

Verification Goal: Purpose: Cover each value of address offset (least significant bit of target address) 
The adress offset containts SET and cachline offset 
Sampling Event: The coverage is sampled when a valid request is received. 
Buckets: At least all sets of the cache=>  offset = i*CL_BYTES where i belongs to [0,HPDCACHE_SETS-1]

Coverage Method: Functional Coverage

## Item 31: Tag coverage

Test Case: Tag coverage

Verification Goal: Purpose: Cover the most significant bit of target address
Sampling Event: The coverage is sampled when a valid request is received. 
Buckets:

Coverage Method: Functional Coverage

## Item 32: Cross coverage between the word and op

Test Case: Cross coverage between the word and op

Verification Goal: Purpose: Cross coverage between word and read/write
Buckets: 3 bits /read and write. 8*2 = 16 buckets

Coverage Method: Functional Coverage

## Item 33: Cross coverage between the set and op

Test Case: Cross coverage between the set and op

Verification Goal: Purpose: Cover that each operation is performed on each SET
Sampling Event: The coverage is sampled when a valid request is recieved. 
Bucket: HPDCACHE_SETS*14

Coverage Method: Functional Coverage

## Item 34: Cross coverage between the set, the size and op

Test Case: Cross coverage between the set, the size and op

Verification Goal: Purpose: For each set cover that each operation is performed for each size. 
Sampling Event: The coverage is sampled when a valid request is recieved. 
Bucket: ??

Coverage Method: Functional Coverage

## Item 35: Cross coverage between the set, word and op

Test Case: Cross coverage between the set, word and op

Verification Goal: Purpose: For each set cover that each operation is performaed for each word. 
Sampling Event: The coverage is sampled when a valid request is recieved. 
Bucket: ?

Coverage Method: Functional Coverage

## Item 36: Address coverage

Test Case: Address coverage

Verification Goal: Purpose: Coverage that each bit of the address signal was enable/disable once, and that the extreme value ( only '0 and only '1) are covered with 1 bucket each. 
Sampling Event: When a valid request is received.  
Buckets: 2xHPDCACHE_PA_WIDTH + 2 (extreme cases)

Coverage Method: Functional Coverage

## Item 37: Mem size check

Sub Feature: The request size and len must be correctly transferred from requestors to the memory write interface. The request size must be correct : READ_SIZE=log2(MEM_DATA_WIDTH/8) and READ_LEN=(CL_WIDTH/MEM_DATA_WIDTH)-1. MAX WRITE_SIZE=log2(WBUF_NWORDS*REQ_WORD_WIDTH /8) and WRITE_LEN=0

Test Case: Mem size check

Verification Goal: Purpose: Check to see if hpdcache send correct size on NoC interface.
Method: This check is performed when a new memory request is observed on the NoC interface

Coverage Method: Functional Coverage

## Item 38: Mem len check

Test Case: Mem len check

Verification Goal: Purpose: Check to see if hpdcache send correct length on NoC interface.
Method: This check is performed when a new memory request is observed on the NoC interface

Coverage Method: Functional Coverage

## Item 39: Size coverage

Test Case: Size coverage

Verification Goal: Purpose: Coverage of each value of this signal 
Buckets: 3 bits => 8 buckets

Coverage Method: Functional Coverage

## Item 40: Length coverage

Test Case: Length coverage

Verification Goal: Purpose: Coverage that each valid value of this signal was covered.
Sampling Event: When a valid request is received. Cover the case of zero load/write/amo 
Buckets: 15, The current cofiguration only have 0x13,1, or 4 for read operation.
It supports only len = 0 for write/AMO operation.

Coverage Method: Functional Coverage

## Item 41: Size coverage

Test Case: Size coverage

Verification Goal: Purpose :Coverage of each value of this signal. Cover for each type of transaction. 
Sampling Event: When a valid request is recieved. 
Buckets: 3 bits => 8 x 13

Coverage Method: Functional Coverage

## Item 42: Mem byte enable Check

Sub Feature: The BE masks must be correctly transferred from requesters to the memory write interface.

Test Case: Mem byte enable Check

Verification Goal: Purpose: Check to see if hpdcache sends correct byte enable on NoC interface in case of a STORE/AMO. 
Method: This check is performed when a new memory request is observed on the NoC interface

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 43: Byte enable coverage

Test Case: Byte enable coverage

Verification Goal: Purpose: The purpose is to see the impact of byte enable disabled or enabled. 
Coverage that each bit of the be signal was enable/disable once, and that the extreme value ( only '0 and only '1) are covered with 1 bucket each.
Cover in the case of responses with or without error. 
Sampling Event: Once a response is recieved 
Buckets: HPDCACHE_REQ_DATA_WIDTH/2*2 + 2(extreme cases)

Coverage Method: Functional Coverage

## Item 44: TID coverage

Test Case: TID coverage

Verification Goal: Purpose: Coverage that every value of the tid signal was covered
Sampling Event: The coverage is sampled when a valid request is recieved. 
Buckets: HPDCACHE_TRANS_ID_WIDTH bits => 2**HPDCACHE_TRANS_ID_WIDTH buckets

Coverage Method: Functional Coverage

## Item 45: Mem_be coverage

Test Case: Mem_be coverage

Verification Goal: Purpose: Coverage that every bit of the mem_be signal were enabled/disabled once
Sampling Event: When a valid response is recieved. 
Buckets: HPDCACHE_MEM_DATA_WIDTH/8*2 + 2

Coverage Method: Functional Coverage

## Item 46: Multiple STORE (different byte enable) followed by LOAD

Test Case: Multiple STORE (different byte enable) followed by LOAD

Verification Goal: Purpose: Cover the case where multiple store at the same address with random byte enable followed by a load at the same address. The purpose of this coverage is to stress the write buffer merging logic. 

Description:  Cover the following sequence: 
W@A BE1, W@A BE2 ….W@A Ben LOAD @A
BE1 !=BE2 != ... Ben 
N = 2:8
buckets: 8

Coverage Method: Functional Coverage

## Item 47: Mem TID Check

Sub Feature: The TID of the request must be correct: All Read TIDs are unique and all Write TIDs are unique

Test Case: Mem TID Check

Verification Goal: Purpose: Check to see if hpdcache send correct TID on NoC interface.
Method: This check is performed when a new memory request is observed on the NoC interface

Coverage Method: Functional Coverage

## Item 48: TID coverage

Test Case: TID coverage

Verification Goal: Purpose: Coverage that every value of the tid signal was covered
Sampling Event: The coverage is sampled when a valid request is recieved. 
Buckets: HPDCACHE_TRANS_ID_WIDTH bits => 2**HPDCACHE_TRANS_ID_WIDTH buckets

Coverage Method: Functional Coverage

## Item 49: TID coverage

Test Case: TID coverage

Verification Goal: Purpose: Coverage that every value of the tid signal was covered
Sampling Event: Cover when a valid request is received. 
Buckets: 2**HPDCACHE_REQ_TRANS_ID_WIDTH buckets

Coverage Method: Functional Coverage

## Item 50: ID coverage

Test Case: ID coverage

Verification Goal: Purpose: Coverage that each value of this signal was covered
Sampling Event: When a valid request is recieved. 
Buckets HPDCACHE_MEM_ID_WIDTH bits => 2**HPDCACHE_MEM_ID_WIDTH buckets ( only if it is possible to cover all values with these buckets)

Coverage Method: Functional Coverage

## Item 51: Mem cacheable check

Sub Feature: The 'cacheable' indication must be transferred from requestors to the memory interface.

Test Case: Mem cacheable check

Verification Goal: Purpose: Check to see if hpdcache send correct cacheable bit on NoC interface.
Method: This check is performed when a new memory request is observed on the NoC interface

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 52: Cache enable  ( cfig_cachectrl.E )

Test Case: Cache enable  ( cfig_cachectrl.E )

Verification Goal: Purpose: Cache Enable - When set to 0, all memory accesses are considered uncacheable
Method: Cover group in the test base 
Sample Event: this coverage is sampled at the end of the test in report phase once the counters are read and verified. 
Buckets: 2

Coverage Method: Functional Coverage

## Item 53: Uncacheable coverage

Test Case: Uncacheable coverage

Verification Goal: Purpose: Coverage of each value of this signal
CACHEABLE
UNCACHEABLE
Sampling Event: The coverage is sampled when a valid request is recieved. 
Buckets: 2

Coverage Method: Functional Coverage

## Item 54: Cross coverage uncacheable and opcode

Test Case: Cross coverage uncacheable and opcode

Verification Goal: Cross coverage between the opcodes and the uncacheable signal. Buckets ?

Coverage Method: Functional Coverage

## Item 55: Command coverage

Test Case: Command coverage

Verification Goal: Purpose: Coverage that each value of this signal was covered x cacheable
MEM_READ
MEM_WRITE
MEM_ATOMIC
RESERVED
Sampling Event: When a valid response is recieved. 
Buckets: 3.

Coverage Method: Functional Coverage

## Item 56: Cacheable coverage

Test Case: Cacheable coverage

Verification Goal: Purpose: Coverage that every value of this signal are covered
Sampling Event: When a valid request is recieved 
Buckets: 2 buckets

Coverage Method: Functional Coverage

## Item 57: Command coverage

Sub Feature: The request type must be relayed correctly

Test Case: Command coverage

Verification Goal: Purpose: Coverage that each value of this signal was covered x cacheable
MEM_READ
MEM_WRITE
MEM_ATOMIC
RESERVED
Sampling Event: When a valid response is recieved. 
Buckets: 3.

Coverage Method: Functional Coverage

## Item 58: Atomic coverage

Test Case: Atomic coverage

Verification Goal: Purpose: Coverage that each value of this signal was covered
ATOMIC_ADD
ATOMIC_CLR
ATOMIC_SET
ATOMIC_EOR
ATOMIC_SMAX
ATOMIC_SMIN
ATOMIC_UMAX
ATOMIC_UMIN
ATOMIC_SWAP
RESERVED
ATOMIC_LDEX
ATOMIC_STEX
Sampling Event: When a valid response is recieved 
Buckets: 11

Coverage Method: Functional Coverage

## Item 59: OP coverage (meaningfull)

Test Case: OP coverage (meaningfull)

Verification Goal: Purpose: Coverage that every (following) operation is followed by every operation on the same address followed by a load on the same addresse.  
STORE
AMO_LR
AMO_SC
AMO_SWAP
AMO_ADD
AMO_AND
AMO_OR
AMO_XOR
AMO_MAX
AMO_MAXU
AMO_MIN
AMO_MINU
CMO
Sampling Event: The coverage is sampled when a valid request is recieved. 
Buckets: Bucket : 13x13 == 169

Coverage Method: Functional Coverage

## Item 60: SID coverage

Sub Feature: SID of the requester must be unique

Test Case: SID coverage

Verification Goal: Purpose: Coverage that every value of the sid signal was covered
Sampling Event: Cover when a valid request is received. 
Buckets: 2**HPDCACHE_REQ_SRC_ID_WIDTH buckets

Coverage Method: Functional Coverage

## Item 61: SID coverage

Test Case: SID coverage

Verification Goal: Purpose: Coverage that every value of the sid signal was covered
Sampling Event: The coverage is sampled when a valid request is recieved. 
Buckets: HPDCACHE_SRC_ID_WIDTHbits => 2**HPDCACHE_SRC_ID_WIDTH buckets

Coverage Method: Functional Coverage

## Item 62: Mem data Check

Sub Feature: Cache mus t send the correct data to the NoC interface in the case of STROE/AMOs

Test Case: Mem data Check

Verification Goal: Purpose: Check to see if hpdcache send correct data on NoC interface in case of a STORE/AMO. 
Method: This check is performed when a new memory request is observed on the NoC interface 
This check is performed only on the case where data merge does not happen

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 63: Wdata coverage

Test Case: Wdata coverage

Verification Goal: Purpose: Coverage that each bit of the wdata signal was enable/disable once, and that the extreme value ( only '0 and only '1) are covered with 1 bucket each.
Sampling Event: When a the response of a write request is recieved. 
Buckets: HPDCACHE_REQ_DATA_WIDTH*2 + 2(extreme cases).

Coverage Method: Functional Coverage

## Item 64 - Requirement 5.0: Number of outstanding Read transactions

Requirement ID: 5.0

Feature: Must support multiple outstanding requests from each requestor.

Sub Feature: Support for multiple outstanding read requests originating from one or multiple requestors.

Feature Description: Must support multiple outstanding requests from each requestor.

Test Case: Number of outstanding Read transactions

Verification Goal: Purpose: The purpose of this coverage is to check the capacity of MSHR in the case of cacheable requests. Coverage on the number of outstanding read request from core. Max MSHR_SETS*MSHR_WAYS+ HPDCACHE_NUM_RTAB miss read and 1 non cacheable. Cover in the case of 1 ou  multiple requesters enabled. 
Sapmling Event: 
Buckets: HPDCACHE_NUM_RTAB+HPDCACHE_MSHR_SET*HPDCACHE_MSHR_WAYS

Coverage Method: Functional Coverage

## Item 65: Back Pressure with load at the same address (offset random)

Test Case: Back Pressure with load at the same address (offset random)

Verification Goal: Purpose :  The purpose of this test is to verify the number of requests accepted by the MSHR and rtab in the case where acceses are made at the consecutive SETS.  The number of accepted requests depends on rtab_single_flag. 
Target:  MSHR and RTAB mecanism 
Scenerio: In this test, the following sequence  is run with memory reponses disabled for first 1000 cycles. We then check the number of requests accepted by the cache, just before the memory reponses are enabled

    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end

Stimuli: 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :  In this test OP is fixed to HPDCACHE_REQ_LOAD. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets  are random.

The maximum number of requests will be actually dependant of min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS, HPDCACHE_SETS). Here you should replace the 8 by RTAB_ENTRIES

Criteria Pass Fail: Criteria (PASS/FAIL) : 
We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  
Expected number of request the moment back pressure is released after 1000 cycles

get_req_counter() == (m_cfg_rtab_single_entry == 1) ? HPDCACHE_SETS + 1: HPDCACHE_SETS +RTAB_ENTRIES

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 66: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of MSHR, RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be NUM_WBUF_ENTRIES.  If the value does not match the expected a UVM_ERROR is flaged. 
Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 67: Back Pressure with load at the same address (offset random)

Test Case: Back Pressure with load at the same address (offset random)

Verification Goal: Purpose :  The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where acceses are made at the consecutive SETS.  The number of accepted requests depends on rtab_single_flag. 
Target:  MSHR and RTAB mecanism 
Scenerio: In this test, the following sequence  is run with memory reponses disabled for first 1000 cycles. We then check the number of requests accepted by the cache, just before the memory reponses are enabled

    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end

Stimuli: 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :  In this test OP is fixed to HPDCACHE_REQ_LOAD. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets  are random.

The maximum number of requests will be actually dependant of min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS, HPDCACHE_SETS). Here you should replace the 8 by RTAB_ENTRIES

Criteria Pass Fail: Criteria (PASS/FAIL) : 
We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  
Expected number of request the moment back pressure is released after 1000 cycles

get_req_counter() == (m_cfg_rtab_single_entry == 1) ? HPDCACHE_SETS + 1: HPDCACHE_SETS +RTAB_ENTRIES

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 68: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES).  If the value does not match the expected a UVM_ERROR is flaged.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 69: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Sub Feature: Support for multiple outstanding write requests from one or multiple requestors

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? HPDCACHE_WBUF_DIR_ENTRIES: 1) +  num_rtab_entry;
        else 
            num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
        end 
        default: 
        begin
          num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES; 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 70: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? HPDCACHE_WBUF_DIR_ENTRIES : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
              default :    num_expected_write =CNT*HPDCACHE_WBUF_DIR_ENTRIES; 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 71: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of MSHR, RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be NUM_WBUF_ENTRIES.  If the value does not match the expected a UVM_ERROR is flaged. 
Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 72: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES): 1) +  num_rtab_entry;
        else 
            num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
        end 
        default: 
        begin
          num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 73: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
              default :    num_expected_write =CNT*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 74: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES).  If the value does not match the expected a UVM_ERROR is flaged.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 75: Write only with limited back pressure

Test Case: Write only with limited back pressure

Verification Goal: Purpose :  The aim of this performance test is to show the benefits of having a write buffer on the performance. 
Check the bandwidth of cached write at the core interface of HPDcache when writes arrive with no inter request delay. 
If memory response reply within a delay < NUM_WBUF cycles, the cache should be able to accept every write. 
Target     : Write Performance 
Scenario:  Write only test. 
Stimuli   :  Test is derived from random access tests.  
Requirement : 
Following condition should be respected to get full performance 
1. Store accesses are cacheable 
2. There is not inter request delay @core interface 
3. Memory response model reply within minimum (< NUM_WBUF cycles) delay possible 

Constraints: 
OP = HPDCACHE_REQ_STORE, uncacheable=0. 
The wbuf and rtab should be configured to get the maximum output. Following values are used:
                 m_cfg_wbuf_threshold = > 2 < HPDCACHE_WBUF_DIR_ENTRIES
                 m_cfg_wbuf_reset_timecnt_on_write = 1;
                 m_cfg_wbuf_sequential_waw = 0;
                 m_cfg_wbuf_inhibit_write_coalescing = 0;
                 m_cfg_rtab_single_entry = 0

All writes are done at unique SET where no request is on the fly.

Criteria Pass Fail: 
Criteria (PASS/FAIL): 
A check is performed at the end of test in report phase.
The cache is expected to accept  NB_TXN stores in NB_TXN cycles. 

In the case where write_buf_dir_entry, write_buf_data_entry < the memory write latency,  almost 3 times degrdation of performance is observed.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 76: Number of outstanding Write transactions

Test Case: Number of outstanding Write transactions

Verification Goal: Purpose: The purpose of this coverage to check the capacity of write buffer in the case of cacheable requests. Coverage on the number of outstanding write request from core. Write MIN(WBUF_DIR_ENTRIES, WBUF_DATA_ENTRIES)  maximum cacheable and 1 non cacheable. Cover in the case of 1 or multiple requesters enabled. 
Sapmling Event: 
Buckets: HPDCACHE_NUM_RTAB+MIN(WBUF_DIR_ENTRIES, WBUF_DATA_ENTRIES)

Coverage Method: Functional Coverage

## Item 77: Write OP, cache operation sequence different addresses

Test Case: Write OP, cache operation sequence different addresses

Verification Goal: Purpose: Cover that an operation at an address that follows a write operation on another address, has no impact on write operation

Description: Write op at address A (byte enable != 0)  followed by each of the following operation at address B (A!= B) followed by Load @ address A
AMO_LR,AMO_SC,AMO_SWAP,AMO_ADD,AMO_AND,AMO_OR,AMO_XOR,AMO_MAX,AMO_MAXU,AMO_MIN,AMO_MINU,CMO(CMO_FENCE, CMO_INVAL_LINE, CMO_INVAL_SET_WAY, CMO_INVAL_ALL, CMO_PREFETCH),LOAD,STORE, CSR READ, CSR WRITE

Write Op is one of the following: 
"STORE  cached, STORE uncached, AMO_LR,AMO_SC,AMO_SWAP,AMO_ADD,AMO_AND,AMO_OR,AMO_XOR,AMO_MAX,AMO_MAXU,AMO_MIN,AMO_MINU, CSR WRITE"
Buckets: 20x14

Coverage Method: Functional Coverage

## Item 78: CSR, cache operation sequence different addresses

Test Case: CSR, cache operation sequence different addresses

Verification Goal: Purpose: Cover that an operation at an address that follows a CSR write operation, has no impact on CSR operation. 

Description:  Cover the following sequence 
CSR write @A followed by any operation followed by CSR read @A. 
Any operation:  AMO_LR,AMO_SC,AMO_SWAP,AMO_ADD,AMO_AND,AMO_OR,AMO_XOR,AMO_MAX,AMO_MAXU,AMO_MIN,AMO_MINU,CMO(CMO_FENCE, CMO_INVAL_LINE, CMO_INVAL_SET_WAY, CMO_INVAL_ALL, CMO_PREFETCH),LOAD,STORE, CSR READ, CSR WRITE @B != A

Coverage Method: Functional Coverage

## Item 79: Write OP, N  other operation sequence different addresses

Test Case: Write OP, N  other operation sequence different addresses

Verification Goal: Purpose: Cover that N operations at an address that follows a write operation on another address, has no impact on write operation. 

Description:  Write op at address A (byte enable != 0)  followed by N operation @B!= A followed by Load @ address A
N = 1:100

Write Op is one of the following: 
"STORE  cached, STORE uncached, AMO_LR,AMO_SC,AMO_SWAP,AMO_ADD,AMO_AND,AMO_OR,AMO_XOR,AMO_MAX,AMO_MAXU,AMO_MIN,AMO_MINU, CSR WRITE"
Buckets: 14x100 (may be too much buckets)

Coverage Method: Functional Coverage

## Item 80: Write OP, cache operation sequence same addresses forbid amo mode

Test Case: Write OP, cache operation sequence same addresses forbid amo mode

Verification Goal: Purpose: Cover the case where  an operations at an address, follows a write operation at same address 
Description:  Write op at address A (byte enable != 0)  followed by each of the following operation at address A followed by Load @ address A
AMO_LR,AMO_SC,AMO_SWAP,AMO_ADD,AMO_AND,AMO_OR,AMO_XOR,AMO_MAX,AMO_MAXU,AMO_MIN,AMO_MINU,CMO(CMO_FENCE, CMO_INVAL_LINE, CMO_INVAL_SET_WAY, CMO_INVAL_ALL, CMO_PREFETCH),LOAD,STORE

Write Op is one of the following: 
"STORE  cached, STORE uncached, AMO_LR,AMO_SC,AMO_SWAP,AMO_ADD,AMO_AND,AMO_OR,AMO_XOR,AMO_MAX,AMO_MAXU,AMO_MIN,AMO_MINU"
Buckets: 20x14

Coverage Method: Functional Coverage

## Item 81: Write OP, cache operation sequence same addresses repilcated amo mode (same as above sequence)

Test Case: Write OP, cache operation sequence same addresses repilcated amo mode (same as above sequence)

Verification Goal: Purpose:

Description:  Write op at address A (byte enable != 0)  followed by each of the following operation at address A followed by Load @ address A
AMO_LR,AMO_SC,AMO_SWAP,AMO_ADD,AMO_AND,AMO_OR,AMO_XOR,AMO_MAX,AMO_MAXU,AMO_MIN,AMO_MINU,CMO(CMO_FENCE, CMO_INVAL_LINE, CMO_INVAL_SET_WAY, CMO_INVAL_ALL, CMO_PREFETCH),LOAD,STORE

Write Op is one of the following: 
"STORE  cached, STORE uncached, AMO_LR,AMO_SC,AMO_SWAP,AMO_ADD,AMO_AND,AMO_OR,AMO_XOR,AMO_MAX,AMO_MAXU,AMO_MIN,AMO_MINU"

Coverage Method: Functional Coverage

## Item 82: Multiple STORE (different byte enable) followed by LOAD

Test Case: Multiple STORE (different byte enable) followed by LOAD

Verification Goal: Purpose: Cover the case where multiple store at the same address with random byte enable followed by a load at the same address. The purpose of this coverage is to stress the write buffer merging logic. 

Description:  Cover the following sequence: 
W@A BE1, W@A BE2 ….W@A Ben LOAD @A
BE1 !=BE2 != ... Ben 
N = 2:8
buckets: 8

Coverage Method: Functional Coverage

## Item 83: Number of store merge

Test Case: Number of store merge

Verification Goal: Purpose: Cover the number of store merge 
Sampling Event: When a store merge is observed 
Method: The congestion tests contains the information regarding the store merge. 
Buckets: ??

Coverage Method: Functional Coverage

## Item 84: Number of outstanding read and write transactions

Sub Feature: Support for multiple outstanding read and write requests originating from one or multiple requestors.

Test Case: Number of outstanding read and write transactions

Verification Goal: Purpose: The purpose of this coverage is to check the capacity of MSHR in the case of cacheable requests. Coverage on the number of outstanding read request from core. Max MSHR_SETS*MSHR_WAYS+ HPDCACHE_NUM_RTAB miss read and 1 non cacheable. Cover in the case of 1 ou  multiple requesters enabled. 
Sapmling Event: 
Buckets: HPDCACHE_NUM_RTAB+HPDCACHE_MSHR_SET*HPDCACHE_MSHR_WAYS

## Item 85: Cross coverage outstanding transaction

Test Case: Cross coverage outstanding transaction

Verification Goal: Purpose: Cross coverage between the number of outstanding read request and the number of outstanding write request
Sampling Event: The coverage is sampled when a valid request is recieved. 
Bucket: For READ: MSHR_SET (in the step of 5), For Write: Write Buf Entries (in the step of 5)

Coverage Method: Functional Coverage

## Item 86: Schmoo 1 (hpdcache req  -> memory req interface)

Test Case: Schmoo 1 (hpdcache req  -> memory req interface)

Verification Goal: Purpose: This schmoo is intended to cover the temporal relationship between a processor operation for a cache line and that cache line address appearing on a AXI request  transaction( from a previous request on the same line). 
Cover 3 cases when NoC ready is zero, random, one during the whole duration of assertion. 

Description:
hpdcache_req_1_valid (ld/st/cmos)  (p_i = 0/1)-> #N mem_req_2_valid  (ld/st)
N = [-5 : 5] @same tag/set
p_i = phys_indexed 
offset between req 1 and req2 have the same or different offsets  
Trigger and clocking events: The clock is core clock. Assertion is trigerred by the request 1 valid. 
Number of Cases: 11x3x2x2x3 = 396 
Note: Covers following cases of on hold request 
1. Cacheable LOAD or PREFETCH, and there is a hit on a pending miss (hit on the MSHR). 
2. Cacheable LOAD or PREFETCH, there is a miss on the cache, and there is a hit (cacheline granularity) on an opened, pending or sent entry of the WBUF 
3. Cacheable STORE, there is a miss on the cache, and there is a hit on a pending miss (hit on the MSHR)
6. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the miss handler FSM cannot send the read miss request

Coverage Method: Assertion Coverage

## Item 87: Schmoo 2  (hpdcache req  -> memory rsp interface)

Test Case: Schmoo 2  (hpdcache req  -> memory rsp interface)

Verification Goal: Purpose: This schmoo is intended to cover the temporal relationship between a processor operation for a cache line and that cache line address appearing on a AXI response  transaction( from a previous request on the same line).
Description: 
hpdcache_req_1_valid(ld/st/cmos)  (p_i = 0/1) -> #N mem_resp_2_valid (ld/st) (error/non error)
N = [-5 : 5] @same tag/set
p_i = phys_indexed 
offset between req 1 and req2 have the same or different offsets
Trigger and clocking events: The clock is core clock. Assertion is trigerred by the request 1 valid. 
Number of Cases: 11x3x2x2x2= 264

Coverage Method: Assertion Coverage

## Item 88: Schmoo 3 (memory req interface -> memory rsp interface)

Test Case: Schmoo 3 (memory req interface -> memory rsp interface)

Verification Goal: Purpose: This schmoo is intended to cover the temporal relationship between an axi operation for a cache line and that cache line address appearing on a AXI request  transaction( from a previous request on the same line).
Description:
mem_req_1_valid(ld/st) -> #N mem_resp_2_valid (ld/st) (error/non error)
N = [-5 : 5] @same tag/set 
offset between req 1 and req2 have the same or different offsets
Trigger and clocking events: The clock is core clock. Assertion is trigerred by the request 1 valid. 
Number of Cases: 11x2x2x2 = 88

Coverage Method: Assertion Coverage

## Item 89: Schmoo 4 (hpdcache req -> hpdcache rsp)

Test Case: Schmoo 4 (hpdcache req -> hpdcache rsp)

Verification Goal: Purpose: This schmoo is intended to cover the temporal relationship between a processor for a cache line and that cache line address appearing on the response interface of hpdcache( from a previous request on the same line).
Description:
hpdcache_req_1_valid(ld/st/cmos)   (p_i = 0/1) -> #N hpdcache_rsp_2_valid (ld/st/cmos) (error/non error)
N = [-5 : 5] @same tag/set 
p_i = phys_indexed 
offset between req 1 and req2 have the same or different offsets
Trigger and clocking events: The clock is core clock. Assertion is trigerred by the request 1 valid. 
Number of Cases: 11x3x2x3x2 = 396

Coverage Method: Assertion Coverage

## Item 90: Shcmoo 5 (hpdcache_req  (@NREQ= X) -> hpdcache_req (@NREQ=Y) )

Test Case: Shcmoo 5 (hpdcache_req  (@NREQ= X) -> hpdcache_req (@NREQ=Y) )

Verification Goal: Purpose: This schmoo is intended to cover the temporal relationship between 2 processor issuing a request at the same time.  If pys_indexed = 0, cache needs 2 cycle to read the address, 1 st cycle it (arbiter) accepts the request and second cycle is used to accept the virtual addresse. 
If another processor makes a request at this second cycle, we have 2 requests coming at the same time. Cover the case where NOC ready always zero, one or random during the duration of assertion
Description:
hpdcache_req_X_valid (ld/st/cmos/amos) (p_i = 0/1) -> #N hpdcache_req_Y_valid (ld/st/cmos/amos)
N = [-5 : 5] 
p_i = phys_indexed 

Trigger and clocking events: The clock is core clock. Assertion is trigerred by the request 1 valid. 
Number of Cases: 11x2x4x4x2 =704

Coverage Method: Assertion Coverage

## Item 91: test_hpdcache_multiple_random_requests

Test Case: test_hpdcache_multiple_random_requests

Verification Goal: Purpose : Test to perform random accesses. The purpose is to have maximum coverage(functional and code).  
Target     : It targets every feature of the cache. 
Scenario: Random
Stimuli : This test  respects the constraints related to hpdcache protocol
Requirement : 
Constraint      :  Every field of the transaction is randomized. Reserved op code is not used. The address are biased to more frequently select the extreme tag values(0x0, 0xFFFFF ….). The distribution is biased to have more request with small delays between requests.
Cacheable requests are previleged over uncacheable requests (95% cacheable / 5% uncacheable). Uncacheable requests empty the pipeline, hence limiting concurrency. Concurrency is the best way to detect possible deadlocks/livelocks.
"Normal" accesses (load, stores) are previliged over CMOs and AMOs (90% normal / 5% CMOs and AMOs)

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 92: test_hpdcache_multiple_random_requests_uncached

Test Case: test_hpdcache_multiple_random_requests_uncached

Verification Goal: This test is derived from test 1.1.1. 
Constraint       :  The field uncacheable is constrained  to have only uncacheable accesses. And is added for coverage.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 93: test_hpdcache_multiple_load_store_requests

Test Case: test_hpdcache_multiple_load_store_requests

Verification Goal: This test is derived from test 1.1.1. 
Stimuli:   In this test 80% of the traffic is load and store.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 94: test_hpdcache_multiple_random_requests_in_region

Test Case: test_hpdcache_multiple_random_requests_in_region

Verification Goal: Purpose : Test to perform random accesses within randomized address  regions. The purpose is to provoke hits and conflicts keeping stimulus relatively random. 
Target     : It targets every feature of the cache. 
Scenario: Random
Stimuli : This test  respects the constraints related to hpdcache protocol
Requirement :  The regions are generated using memory partition class from cv_dv_utils. The regions are random size but some are small which provokes hits and conflicts.
The sequence choses one region randomly for a SEED.
Constraint       :  Every field of the transaction is randomized. Reserved op code is not used. The addresses are generated within the region. The distribution is biased to have more request with small delays between requests.
Cacheable requests are previleged over uncacheable requests (95% cacheable / 5% uncacheable). Uncacheable requests empty the pipeline, hence limiting concurrency. Concurrency is the best way to detect possible deadlocks/livelocks.
"Normal" accesses (load, stores) are previliged over CMOs and AMOs (90% normal / 5% CMOs and AMOs)

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 95: test_hpdcache_multiple_load_store_requests_in_region

Test Case: test_hpdcache_multiple_load_store_requests_in_region

Verification Goal: Same as 2.1.4. 
Stimuli: This test performs  80% load store accesses within randomized address regions and the other accesses are fully random but within the regions.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 96: Write(30%) Read(70% ) with no memory back pressure

Test Case: Write(30%) Read(70% ) with no memory back pressure

Verification Goal: Purpose : Check the bandwidth of cached reads (70%) and cached writes(30%) at the core interface of HPDcache when reads and writes arrive with no inter request delay. 
Target     : Read and Write Performance 
Scenario:  Test is constraint to have 70% cacheable read and 30% cacheable  write. 
Stimuli   :  Test is derived from random access tests.  
Requirements: 
Following condition are respected to get full performance 
1. Load/Store accesses are cacheable 
2. There is no conflit within the MSHR (An access should not be made on a SET which already has an ongoing miss) 
3. There is no inter request delay @core interface 
4. Memory response model reply within minimum delay possible 
Constraints: 
OP = HPDCACHE_REQ_LOAD(70%), HPDCACHE_REQ_STORE(30%). Uncacheable = 0. 
5. The wbuf and rtab should be configurered to get the maximum outuput. Following values are used:
                 m_cfg_wbuf_threshold = > 2
                 m_cfg_wbuf_reset_timecnt_on_write = 1;
                 m_cfg_wbuf_sequential_waw = 0;
                 m_cfg_wbuf_inhibit_write_coalescing = 0;
                 m_cfg_rtab_single_entry = 0

Criteria Pass Fail: A check is performed at the end of test in report phase. 
The cache is expected to accept  NB_TXN loads/stores in 3*NB_TXN cycles.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 97 - Requirement 6.0: Size coverage

Requirement ID: 6.0

Feature: Can provide 1 to 64 bytes of a cacheline per cycle

Sub Feature: Any given requester can access 1 to 64 bytes of a cacheline per cycle

Feature Description: Can provide 1 to 64 bytes of a cacheline per cycle

Test Case: Size coverage

Verification Goal: Purpose: Coverage of each value of this signal 
Buckets: 3 bits => 8 buckets

Coverage Method: Functional Coverage

## Item 98: Mem_data coverage

Test Case: Mem_data coverage

Verification Goal: Purpose: Coverage that each bit of the mem_data signal was enable/disable once, and that the extreme value ( only '0 and only '1) are covered with 1 bucket each. 
Sampling Event: When a valid response is recieved. 
Buckets: HPDCACHE_MEM_DATA_WIDTH*2 + 2

Coverage Method: Functional Coverage

## Item 99 - Requirement 7.0: Size coverage

Requirement ID: 7.0

Feature: Cache must provide correct responses to requesters.

Sub Feature: Provide 1 to 64 bytes of data, based on request size.

Feature Description: Cache must provide correct responses to requesters.

Test Case: Size coverage

Verification Goal: Purpose: Coverage of each value of this signal 
Buckets: 3 bits => 8 buckets

Coverage Method: Functional Coverage

## Item 100: Rdata coverage

Test Case: Rdata coverage

Verification Goal: Purpose: Coverage that each bit of the Rdata signal was enable/disable once, and that the extreme value ( only '0 and only '1) are covered with 1 bucket each. 
Sampling Event: Cover when a valid LOAD/AMO/LR response is received. 
Buckes: HPDCACHE_REQ_DATA_WIDTH*2 buckets  + 2 (extreme values)

Coverage Method: Functional Coverage

## Item 101: Last coverage

Test Case: Last coverage

Verification Goal: Purpose:Coverage that each values of this signal are covered
Bucket: 2 buckets
Samlng Event: Sample at the time of NoC Reponse . Sample when the NoC response is witout an error. Sample at the time of NoC Reponse . Sample when the NoC response is witout an error.

Coverage Method: Functional Coverage

## Item 102: Unsolicited Response

Sub Feature: The TID of the response must correspond to the TID of the original request.

Test Case: Unsolicited Response

Verification Goal: Purpose: Check if cache send an unsolicited response. Cache shall ignore this unsolicited response. 
Method: The memory response model is used to insert unsolicited responses.
NOTE: Not sure how this test works. If the memory send a response with no matching request, the behavior of the HPDcache is somehow undefined. There is no countermeasure for unsolicited responses. If it is a read response, with an ID different than '1 (all bits to 1), the HPDcache will probably forward a response to the core. If all bits of the ID are set to 1, the HPDcache will forward the response on the next uncacheable request

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 103: Unsolicited Response

Sub Feature: The SID of the response must correspond to the SID of the original requester.

Test Case: Unsolicited Response

Verification Goal: Purpose: Check if cache send an unsolicited response. Cache shall ignore this unsolicited response. 
Method: The memory response model is used to insert unsolicited responses.
NOTE: Not sure how this test works. If the memory send a response with no matching request, the behavior of the HPDcache is somehow undefined. There is no countermeasure for unsolicited responses. If it is a read response, with an ID different than '1 (all bits to 1), the HPDcache will probably forward a response to the core. If all bits of the ID are set to 1, the HPDcache will forward the response on the next uncacheable request

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 104: SID coverage

Test Case: SID coverage

Verification Goal: Purpose: Coverage that every value of the sid signal was covered
Sampling Event: Cover when a valid request is received. 
Buckets: 2**HPDCACHE_REQ_SRC_ID_WIDTH buckets

Coverage Method: Functional Coverage

## Item 105: Data Response Check

Sub Feature: The cache must always provide coherent data if error is not set.

Test Case: Data Response Check

Verification Goal: Purpose: Check if HPDcache responds with a correct data
Check Data response in following cases 
1. LOAD 
2. SC 
3. AMO 
This check is ignored if data was corrupted by a previous error response from memory and is not overwritten by a fresh data. This line is considered to be corrupted. 
This check is performed when the cache response is received
Method:  A memory shadow is mantained in the HPDcache SB, which is used to predict the correct responses.

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 106: Data coverage

Test Case: Data coverage

Verification Goal: Purpose: Coverage that each bit of the data signal was enable/disable once, and that the extreme value ( only '0 and only '1) are covered with 1 bucket each. 
Bucket: HPDCACHE_MEM_DATA_WIDTH*2 + 2
Sampling Event: Sample at the time of NoC Reponse . Sample when the NoC response is witout an error.

Coverage Method: Functional Coverage

## Item 107: Need_rsp coverage

Sub Feature: The cache must always provide a  response when signal need_rsp is true for a request.

Test Case: Need_rsp coverage

Verification Goal: Purpose: Coverage of each value of this signal
Sampling Event: The coverage is sampled when a valid request is received. 
Buckets: 2 buckets

Coverage Method: Functional Coverage

## Item 108: Cross coverage need_rsp and opcode (load/store)

Test Case: Cross coverage need_rsp and opcode (load/store)

Verification Goal: Purpose: Cross coverage between the opcodes LOAD/STORE and need_rsp signal. 
Sampling Event: The coverage is sampled when a valid request is recieved. 
Buckets: Buckets = 4

Coverage Method: Functional Coverage

## Item 109: Data Response Check

Test Case: Data Response Check

Verification Goal: Purpose: Check if HPDcache responds with a correct data
Check Data response in following cases 
1. LOAD 
2. SC 
3. AMO 
This check is ignored if data was corrupted by a previous error response from memory and is not overwritten by a fresh data. This line is considered to be corrupted. 
This check is performed when the cache response is received
Method:  A memory shadow is mantained in the HPDcache SB, which is used to predict the correct responses.

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 110: Unsolicited Response

Sub Feature: The cache must never provide a  response when signal need_rsp is false for a request.

Test Case: Unsolicited Response

Verification Goal: Purpose: Check if cache send an unsolicited response. Cache shall ignore this unsolicited response. 
Method: The memory response model is used to insert unsolicited responses.
NOTE: Not sure how this test works. If the memory send a response with no matching request, the behavior of the HPDcache is somehow undefined. There is no countermeasure for unsolicited responses. If it is a read response, with an ID different than '1 (all bits to 1), the HPDcache will probably forward a response to the core. If all bits of the ID are set to 1, the HPDcache will forward the response on the next uncacheable request

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 111: Error Response Check

Sub Feature: If an error occurs during a response, the cache must respond with the signal error is set for that response except in the case of caceahble write.

Test Case: Error Response Check

Verification Goal: Purpose :  The error responses are generated correctly. 
Description: The error is inserted on the NoC side by memory response model. In the case of error, the cache send an error response. Except in the case of store 
This check if performed when the cache response is received

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 112: Load with Error response

Test Case: Load with Error response

Verification Goal: Purpose: Cover that a Load (miss) with an error response (memory)  is followed by a LOAD/AMO (no rsp ) on the same line. Cover for cache and uncached 
transactions.
Trigger and Clocking Events: The clock is a core clk. The assertion is triggered by when a valid load is recieved.

Coverage Method: Functional Coverage

## Item 113: Error coverage

Test Case: Error coverage

Verification Goal: Purpose: Coverage of each value of this signal
Sampling Event: Cover when a valid request is received. 
Buckets:  bit => 2 buckets

Coverage Method: Functional Coverage

## Item 114: Error coverage

Test Case: Error coverage

Verification Goal: Purpose: Error reponse from NoC. 
Coverage that each values of this signal are covered 
Buckets: 2 buckets
Sampling Event: Sample at the time of NoC Reponse

Coverage Method: Functional Coverage

## Item 115: W_is_atomic coverage

Test Case: W_is_atomic coverage

Verification Goal: Purpose: Coverage that each values of this signal are covered
Bucket: 2 buckets
Sampling Event: Sample at the time of NoC Reponse . Sample when the NoC response is witout an error.

Coverage Method: Functional Coverage

## Item 116: W_error coverage

Test Case: W_error coverage

Verification Goal: Purpose: Coverage that each values of this signal are covered
Bucket: 
MEM_RESP_OK
MEM_RESP_NOK
(There is two possibles value that are not defined : are they RESERVED ?)
Sampling Event: Sample at the time of NoC Reponse . Sample when the NoC response is witout an error.

Coverage Method: Functional Coverage

## Item 117: Features not part of test plan

Sub Feature: The cache must operate correctly, even if requesters re-use TID values for multiple, in-flight requests.

Test Case: Features not part of test plan

## Item 118 - Requirement 8.0: Uncacheable coverage

Requirement ID: 8.0

Feature: The cache must handle cacheable and non-cacheable requests.

Sub Feature: The cache determines whether a given requets is cacheable based on the UNCACHEABLE signal.

Feature Description: The cache must handle cacheable and non-cacheable requests.

Test Case: Uncacheable coverage

Verification Goal: Purpose: Coverage of each value of this signal
CACHEABLE
UNCACHEABLE
Sampling Event: The coverage is sampled when a valid request is recieved. 
Buckets: 2

Coverage Method: Functional Coverage

## Item 119: test_hpdcache_multiple_random_requests

Test Case: test_hpdcache_multiple_random_requests

Verification Goal: Purpose : Test to perform random accesses. The purpose is to have maximum coverage(functional and code).  
Target     : It targets every feature of the cache. 
Scenario: Random
Stimuli : This test  respects the constraints related to hpdcache protocol
Requirement : 
Constraint      :  Every field of the transaction is randomized. Reserved op code is not used. The address are biased to more frequently select the extreme tag values(0x0, 0xFFFFF ….). The distribution is biased to have more request with small delays between requests.
Cacheable requests are previleged over uncacheable requests (95% cacheable / 5% uncacheable). Uncacheable requests empty the pipeline, hence limiting concurrency. Concurrency is the best way to detect possible deadlocks/livelocks.
"Normal" accesses (load, stores) are previliged over CMOs and AMOs (90% normal / 5% CMOs and AMOs)

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 120: test_hpdcache_multiple_directed_addr

Test Case: test_hpdcache_multiple_directed_addr

Verification Goal: Purpose :  Test to provoke the eviction at regular interval in between many hits.
Target     :  This test stresses the eviction mechanism. 
Scenario:  In this test, one set is selected randomly at the beginning of the test.  Then NWAYS + 1 random tags are selected within the selected set. Random loads  and stores are performed on these preselected {tag, set}. Normally we should observe many hits with misses (because of the evictions) as the number of tags exceed the NWAYS by 1.  
Stimuli :  The test runs hpdcache_multiple_directed_addr sequence which is driven from hpdcache_single_directed_addr. 
Requirement :  This test  respects the constraints related to hpdcache protocol
Constraint       :  Every field of the transaction is randomized except set, address and uncacheable (always 0 -> cacheable).

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 121: test_hpdcache_multiple_directed_addr_bPLRU_prediction

Test Case: test_hpdcache_multiple_directed_addr_bPLRU_prediction

Verification Goal: Purpose:  The purpose of this the test is to check the PLRU eviction algorithm using a black box approach.
Target:  PLRU algorithm 
Scenario:  To avoid temporal conflicts between request and refill response which cannot be predicted, in this test each request is sent every N( = 15) cycles and the memory response model is configured to respond with zero delay. We require that the response from the previous request is received before the next request is issued. Otherwise it is impossible to predict the PLRU behavior using a black box approach.
Stimuli :  The sequence is derived from 1.2.1
Requirement :  This test  respects the constraints related to hpdcache protocol. This test enables a UVM based model in the SB which  does the black box prediction of the PLRU under a limited set of stimulus which is respected by this test.
Constraint       :  Every field of the transaction is randomized except Set, address, uncacheable (always 0 -> cacheable) and inter request delay. Memory response model is configured to respond with zero delay. 
Note:  We note that the environment also has a SV based PLRU model which is bind directly to the memory control module to do the exact prediction of the PLRU using the signals from memory control module (white box assertion based checking).   In the UVM based model, the PLRU prediction takes into accoun the type of transactions.

Criteria Pass Fail: The TB has UVM based PLRU model which flags a UVM_ERROR in case PLRU algorithm is not respected. A tag directory is maintained which helps predict hit and miss in this particular scenario.

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 122: Features not part of test plan

Sub Feature: The requestor must be consistent regarding the cachability of a line in the cache.

Test Case: Features not part of test plan

## Item 123 - Requirement 9.0: For address-overlapping transactions, the cache guarantees that these are committed in the order in which they are consumed from the requesters.

Requirement ID: 9.0

Feature: Provide a memory model compliant with RISC-V Weak Memory Ordering (RVWMO)

Sub Feature: If two transactions have overlapping address ranges, then they must be comitted in the order they were issued.

Feature Description: Provide a memory model compliant with RISC-V Weak Memory Ordering (RVWMO)

Test Case: For address-overlapping transactions, the cache guarantees that these are committed in the order in which they are consumed from the requesters.

Verification Goal: Purpose: At any given time, There can never be two inflight requests (read or writes) with overlapping addresses. By the way, the addresses do not need to be identical. The addressed segment cannot overlap: (A.address, A.address + 2**A.size) do not overlap with (B.address, B.address + 2**B.size)
one assertion per byte
Method: System Verilog Temporal Assertion check 
Link: hpdcache_sva.sv

Coverage Method: Assertion Coverage

## Item 124: test_hpdcache_multiple_random_requests

Sub Feature: If two transactions do not have overlapping address ranges, there is no constraint on the order they are committed.

Test Case: test_hpdcache_multiple_random_requests

Verification Goal: Purpose : Test to perform random accesses. The purpose is to have maximum coverage(functional and code).  
Target     : It targets every feature of the cache. 
Scenario: Random
Stimuli : This test  respects the constraints related to hpdcache protocol
Requirement : 
Constraint      :  Every field of the transaction is randomized. Reserved op code is not used. The address are biased to more frequently select the extreme tag values(0x0, 0xFFFFF ….). The distribution is biased to have more request with small delays between requests.
Cacheable requests are previleged over uncacheable requests (95% cacheable / 5% uncacheable). Uncacheable requests empty the pipeline, hence limiting concurrency. Concurrency is the best way to detect possible deadlocks/livelocks.
"Normal" accesses (load, stores) are previliged over CMOs and AMOs (90% normal / 5% CMOs and AMOs)

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 125: READ WRITE at the same address with Back Pressure

Sub Feature: If a cache-able READ or pre-fetch misses in the cache and hits in the write buffer, the write operation must complete to memory before the READ is processed.

Test Case: READ WRITE at the same address with Back Pressure

Verification Goal: We cannot have same addresse at the same time in the MSHR and Write Buffer 
in this test aim to fill write buffer and mshr before RTAB is full.

## Item 126: READ WRITE at the same address with Back Pressure

Sub Feature: If a cache-able STORE misses on the cache but matches a pending miss, the pending miss must complete before issuing the store.

Test Case: READ WRITE at the same address with Back Pressure

Verification Goal: We cannot have same addresse at the same time in the MSHR and Write Buffer 
in this test aim to fill write buffer and mshr before RTAB is full.

## Item 127: READ WRITE at the same address with Back Pressure

Sub Feature: If cacheable operation T1 (read or write) is issued before T2 (read or write) and T1 overlaps T2 then T1 must be issued on the memory interface before T2. This should, of course, be true for the specific cases when T1 and T2 are both writes, T1 and T2 are both reads.

Test Case: READ WRITE at the same address with Back Pressure

Verification Goal: We cannot have same addresse at the same time in the MSHR and Write Buffer 
in this test aim to fill write buffer and mshr before RTAB is full.

## Item 128 - Requirement 10.0: Ready Valid Protocol  (request)

Requirement ID: 10.0

Feature: Provide an 5-channel interface to memory.

Sub Feature: Provide a memory read request interface with ready/valid handshake

Feature Description: Provide an 5-channel interface to memory.

Test Case: Ready Valid Protocol  (request)

Verification Goal: Purpose: Cover following scenerios of ready valid protocol for read and write interfaces (data and meta data). 
1. Read and Valid are inserted at the same time 
2. Ready is high before the valid is inserted 
3. Valid is high before the ready in inserted 
Sampling Event: When a valid request is recieved. 
Buckets: Total 6 scenarios for 2 different interfaces.

## Item 129: Ready Valid Protocol  (response)

Sub Feature: Provide a memory read response interface with ready/valid handshake

Test Case: Ready Valid Protocol  (response)

Verification Goal: Purpose: Cover following scenerios of ready valid protocol for read and write response interfaces 
1. Read and Valid are inserted at the same time 
2. Ready is high before the valid is inserted 
3. Valid is high before the ready in inserted 
Sampling Event: When a valid request is recieved. 
Buckets: Total 6 scenarios for 2 different interfaces.

## Item 130: Ready Valid Protocol  (request)

Sub Feature: Provide a memory write attribute request channel with ready/valid handshake

Test Case: Ready Valid Protocol  (request)

Verification Goal: Purpose: Cover following scenerios of ready valid protocol for read and write interfaces (data and meta data). 
1. Read and Valid are inserted at the same time 
2. Ready is high before the valid is inserted 
3. Valid is high before the ready in inserted 
Sampling Event: When a valid request is recieved. 
Buckets: Total 6 scenarios for 2 different interfaces.

## Item 131: Ready Valid Protocol  (request)

Sub Feature: Provide a memory write-data request channel with ready/valid handshake

Test Case: Ready Valid Protocol  (request)

Verification Goal: Purpose: Cover following scenerios of ready valid protocol for read and write interfaces (data and meta data). 
1. Read and Valid are inserted at the same time 
2. Ready is high before the valid is inserted 
3. Valid is high before the ready in inserted 
Sampling Event: When a valid request is recieved. 
Buckets: Total 6 scenarios for 2 different interfaces.

## Item 132: Ready Valid Protocol  (response)

Sub Feature: Provide a memory write response channel with ready/valid handshake

Test Case: Ready Valid Protocol  (response)

Verification Goal: Purpose: Cover following scenerios of ready valid protocol for read and write response interfaces 
1. Read and Valid are inserted at the same time 
2. Ready is high before the valid is inserted 
3. Valid is high before the ready in inserted 
Sampling Event: When a valid request is recieved. 
Buckets: Total 6 scenarios for 2 different interfaces.

## Item 133 - Requirement 11.0: Ready Valid Protocol  (request)

Requirement ID: 11.0

Feature: Respect the ready/valid protocol on all requester and memory interfaces

Sub Feature: Once VALID is set to '1' it must stay at '1' until RDY goes to '1' and the transfer occurs.

Feature Description: Respect the ready/valid protocol on all requester and memory interfaces

Test Case: Ready Valid Protocol  (request)

Verification Goal: Purpose: Cover following scenerios of ready valid protocol for read and write interfaces (data and meta data). 
1. Read and Valid are inserted at the same time 
2. Ready is high before the valid is inserted 
3. Valid is high before the ready in inserted 
Sampling Event: When a valid request is recieved. 
Buckets: Total 6 scenarios for 2 different interfaces.

## Item 134: Ready Valid Protocol  (response)

Test Case: Ready Valid Protocol  (response)

Verification Goal: Purpose: Cover following scenerios of ready valid protocol for read and write response interfaces 
1. Read and Valid are inserted at the same time 
2. Ready is high before the valid is inserted 
3. Valid is high before the ready in inserted 
Sampling Event: When a valid request is recieved. 
Buckets: Total 6 scenarios for 2 different interfaces.

## Item 135 - Requirement 12.0: test_hpdcache_multiple_amo_lr_sc_requests

Requirement ID: 12.0

Feature: The cache must support 11 AMO operations.

Sub Feature: HPDCACHE_REQ_AMO_LR 
HPDCACHE_REQ_AMO_SC 
HPDCACHE_REQ_AMO_SWAP 
HPDCACHE_REQ_AMO_ADD 
HPDCACHE_REQ_AMO_AND 
HPDCACHE_REQ_AMO_OR 
HPDCACHE_REQ_AMO_XOR 
HPDCACHE_REQ_AMO_MAX 
HPDCACHE_REQ_AMO_MAXU 
HPDCACHE_REQ_AMO_MIN 
HPDCACHE_REQ_AMO_MINU

Feature Description: The cache must support 11 AMO operations.

Test Case: test_hpdcache_multiple_amo_lr_sc_requests

Verification Goal: Purpose: The aim of this test is to test Load Reserved (LR)/Store Conditional (SC) sequences. The goal is to test all possible conditions where LR/SC fails or passes. A coverage item is added to cover all possible combination. A sequence of LR/SC mixed with LOAD/STORE/AMOS/CMOS are run. Most(90%) of the accesses are done at the same address.
Target: Load Reserved and Store Conditional Mechanism 
Scenario:  The hpdcache_single_lr_sc_request runs a sequence with following constraints: 
FOR 1 to (random 10 or 15 )
          m_req_addr dist { same_addr := (50 a 100), random_addr := (100 - same_addr)};
          m_req_op   dist { hpdcache_REQ_AMO_LR := 40, HPDCACHE_REQ_AMO_SC := 40, 
                            hpdcache_REQ_LOAD     := 11,
                            hpdcache_REQ_AMO_SWAP := 1,
                            hpdcache_REQ_STORE  := 1, 
                            hpdcache_REQ_AMO_ADD  := 1,
                            hpdcache_REQ_AMO_AND  := 1,
                            hpdcache_REQ_AMO_OR   := 1,
                            hpdcache_REQ_AMO_XOR  := 1,
                            hpdcache_REQ_AMO_MAX  := 1,
                            hpdcache_REQ_AMO_MAXU := 1,
                            hpdcache_REQ_AMO_MIN  := 1,
                            hpdcache_REQ_AMO_MINU := 1
                            hpdcache_REQ_CMO := 1 
END
Stimuli: The sequence used is hpdcache_multiple_amo_lr_sc_requests which is driven from hpdcache_single_lr_sc_request.
Requirement :  This test  respects the constraints related to hpdcache protocol. 
Constraint:  Every field of the transaction is randomized except address.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 136: OP coverage (meaningfull)

Test Case: OP coverage (meaningfull)

Verification Goal: Purpose: Coverage that every (following) operation is followed by every operation on the same address followed by a load on the same addresse.  
STORE
AMO_LR
AMO_SC
AMO_SWAP
AMO_ADD
AMO_AND
AMO_OR
AMO_XOR
AMO_MAX
AMO_MAXU
AMO_MIN
AMO_MINU
CMO
Sampling Event: The coverage is sampled when a valid request is recieved. 
Buckets: Bucket : 13x13 == 169

Coverage Method: Functional Coverage

## Item 137: Scenarios to cover LR/SC 1

Sub Feature: The HPDC always treats LR/SC operations with a  size of 8 bytes, aligned to 8-byte boundary.

Test Case: Scenarios to cover LR/SC 1

Verification Goal: Purpose: Cover following scenerio of LR/SC
LR(@A(Address), @W(word(8 bytes)) -> SC(@A, @W) -> LOAD(@A, @W) : SC Pass
LR(@A(Address), @W(word) -> SC(@A, @W1 != W) -> LOAD(@A, @W) -> LOAD(@A, @W1) -> SC Fail
LR(@A(Address), @W(word) -> SC(@A1 != A, X) -> LOAD(@A) -> LOAD(@A1) -> SC Fail
LR(@A(Address), @W(word) -> LOAD (@A, @W) -> SC(@A, @W) -> LOAD(@A)  -> SC Pass
LR(@A(Address), @W(word) -> LOAD (@A1 != A) -> SC(@A, @W) -> LOAD(@A)  -> SC Pass
LR(@A(Address), @W(word) -> STORE/AMO (@A, @W1 != W) -> SC(@A, @W) -> LOAD(@A)  -> SC Pass
LR(@A(Address), @W(word) -> STORE/AMO (@A1) -> SC(@A, @W) -> LOAD(@A)  -> SC Pass
LR(@A(Address), @W(word) -> STORE/AMO (@A) -> SC(@A, @W) -> LOAD(@A)  -> SC Fail

Sampling Event: When a SC pass fail is detected by SB. 
Buckets:  8

Coverage Method: Assertion Coverage

## Item 138: Scenarios to cover LR/SC 1

Sub Feature: If the LR is not aligned to an 8-byte boundary, the reservation set can span two 8-byte blocks.

Test Case: Scenarios to cover LR/SC 1

Verification Goal: Purpose: Cover following scenerio of LR/SC
LR(@A(Address), @W(word(8 bytes)) -> SC(@A, @W) -> LOAD(@A, @W) : SC Pass
LR(@A(Address), @W(word) -> SC(@A, @W1 != W) -> LOAD(@A, @W) -> LOAD(@A, @W1) -> SC Fail
LR(@A(Address), @W(word) -> SC(@A1 != A, X) -> LOAD(@A) -> LOAD(@A1) -> SC Fail
LR(@A(Address), @W(word) -> LOAD (@A, @W) -> SC(@A, @W) -> LOAD(@A)  -> SC Pass
LR(@A(Address), @W(word) -> LOAD (@A1 != A) -> SC(@A, @W) -> LOAD(@A)  -> SC Pass
LR(@A(Address), @W(word) -> STORE/AMO (@A, @W1 != W) -> SC(@A, @W) -> LOAD(@A)  -> SC Pass
LR(@A(Address), @W(word) -> STORE/AMO (@A1) -> SC(@A, @W) -> LOAD(@A)  -> SC Pass
LR(@A(Address), @W(word) -> STORE/AMO (@A) -> SC(@A, @W) -> LOAD(@A)  -> SC Fail

Sampling Event: When a SC pass fail is detected by SB. 
Buckets:  8

Coverage Method: Assertion Coverage

## Item 139: Scenarios to cover LR/SC 2 (multiple LR)

Sub Feature: The HPDC supports a single reservation set. If multiple AMO_LR operations are performed, the reservation set is determined by the last one.

Test Case: Scenarios to cover LR/SC 2 (multiple LR)

Verification Goal: Same as 9.1.2
In the case there muliple LR at different addresse before LR@A, @W
HPDcache shall take into account only the last reservation set.

## Item 140: Mem STEX

Sub Feature: An SC operation is forwarded to memory IFF the addressed bytes overlap the reservation set.

Test Case: Mem STEX

Verification Goal: Purpose: Check to verify if SC is passed to memory interface IFF SC is a pass. 
Method: This check is performed when a new memory request is observed on the NoC interface

Coverage Method: Functional Coverage

## Item 141: Scenarios to cover LR/SC 2 (multiple SC)

Sub Feature: An SC operation must clear the reservation, whether it succeeds, or not.

Test Case: Scenarios to cover LR/SC 2 (multiple SC)

Verification Goal: Same as 9.1.2
In the case there mulipleSC @A, @W following LR operation @A, @W
HPDcache shall pass only the first SC operation if it is a pass

## Item 142: Scenarios to cover LR/SC 1

Sub Feature: If a SC accesses a subset of the space of the reservation set, the store  operation is still forwarded to the memory.

Test Case: Scenarios to cover LR/SC 1

Verification Goal: Purpose: Cover following scenerio of LR/SC
LR(@A(Address), @W(word(8 bytes)) -> SC(@A, @W) -> LOAD(@A, @W) : SC Pass
LR(@A(Address), @W(word) -> SC(@A, @W1 != W) -> LOAD(@A, @W) -> LOAD(@A, @W1) -> SC Fail
LR(@A(Address), @W(word) -> SC(@A1 != A, X) -> LOAD(@A) -> LOAD(@A1) -> SC Fail
LR(@A(Address), @W(word) -> LOAD (@A, @W) -> SC(@A, @W) -> LOAD(@A)  -> SC Pass
LR(@A(Address), @W(word) -> LOAD (@A1 != A) -> SC(@A, @W) -> LOAD(@A)  -> SC Pass
LR(@A(Address), @W(word) -> STORE/AMO (@A, @W1 != W) -> SC(@A, @W) -> LOAD(@A)  -> SC Pass
LR(@A(Address), @W(word) -> STORE/AMO (@A1) -> SC(@A, @W) -> LOAD(@A)  -> SC Pass
LR(@A(Address), @W(word) -> STORE/AMO (@A) -> SC(@A, @W) -> LOAD(@A)  -> SC Fail

Sampling Event: When a SC pass fail is detected by SB. 
Buckets:  8

Coverage Method: Assertion Coverage

## Item 143: Mem Access Check

Sub Feature: All atomic operations must be sent to the write-request interface.

Test Case: Mem Access Check

Verification Goal: Purpose: Check to see if hpdcache makes a correct type of memory request on NoC interface. 
Method: This check is performed when a new memory request is observed on the NoC interface

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 144: Mem AMO check

Sub Feature: The cache must process two responses from the memory for each atomic operation : old_data on read response channel and write acknowledge on memory write response channel.

Test Case: Mem AMO check

Verification Goal: Purpose: Check to verify if AMO reqest on memory write interface has 2 responses: old_data on read response channel and write acknowledge on memory write response channel.
Method: This check is performed when a new memory respones(read and write) are observed on the NoC interface

Coverage Method: Functional Coverage

## Item 145 - Requirement 12.0: Error Response Check

Requirement ID: 12.0

Feature: The cache must support 11 AMO operations.

Sub Feature: HPDCACHE_REQ_AMO_LR 
HPDCACHE_REQ_AMO_SC 
HPDCACHE_REQ_AMO_SWAP 
HPDCACHE_REQ_AMO_ADD 
HPDCACHE_REQ_AMO_AND 
HPDCACHE_REQ_AMO_OR 
HPDCACHE_REQ_AMO_XOR 
HPDCACHE_REQ_AMO_MAX 
HPDCACHE_REQ_AMO_MAXU 
HPDCACHE_REQ_AMO_MIN 
HPDCACHE_REQ_AMO_MINU

Feature Description: The cache must support 11 AMO operations.

Test Case: Error Response Check

Verification Goal: Purpose :  The error responses are generated correctly. 
Description: The error is inserted on the NoC side by memory response model. In the case of error, the cache send an error response. Except in the case of store 
This check if performed when the cache response is received

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 146: SC Data Check

Sub Feature: In the case of the STEX operation, the write acknowledge channel indicates if the write was atomic (mem_resp_w_is_atomic). The HPDC returns the status on the signals HPDCACHE_RSP_DATA (0x0000_0000 = success, 0x0000_0001 = failure). This value is zero extended, if the requested size >= 8 bytes , and replicated if there is more than one word.

Test Case: SC Data Check

Verification Goal: Purpose: Check if Store Conditional responds with the correct data
Description: In the case of SC FAIL/PASS, data response is checked. The modeling of LR/SC is done in SB. 
In case of SC PASS: Data is 0 
In case of SC FAIL: Data > 0 
This check is performed when the cache response is received 
Method: SB predicts the behaviour of SC response

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 147: SC Data Check

Sub Feature: In case of a hit in the cache on a SC, and that the STEX is atomic, the cache updates the local data.

Test Case: SC Data Check

Verification Goal: Purpose: Check if Store Conditional responds with the correct data
Description: In the case of SC FAIL/PASS, data response is checked. The modeling of LR/SC is done in SB. 
In case of SC PASS: Data is 0 
In case of SC FAIL: Data > 0 
This check is performed when the cache response is received 
Method: SB predicts the behaviour of SC response

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 148 - Requirement 13.0: Features not part of test plan

Requirement ID: 13.0

Feature: Requester interface must support CMO and fence operations.

Sub Feature: Support for CMO operation must be compatible with definitions in RISC-V CMO standard.

Feature Description: Requester interface must support CMO and fence operations.

Test Case: Features not part of test plan

## Item 149: Cross coverage opcode (meaningfull CMO) and size

Sub Feature: Must support a memory write fence (HPDCACHE_CMO_FENCE). This operation must ensure that all open entries in the write buffer are immediately closed. No new requests are accepted from any requester until all pending write requests have been acknowledged on the NoC interface.

Test Case: Cross coverage opcode (meaningfull CMO) and size

Verification Goal: Purpose: Cross coverage between the opcode CMO and the size signal to cover each CMO operation. Cover the following sequence with every CMO transaction. (Write/Read(cacheable) -> CMO -> Read). The whole sequence should be on the same address.  This should be covered for each line. 
CMO_FENCE
CMO_INVAL_INLINE
CMO_INVAL_ALL
CMO_PREFETCH
Sampling Event: The coverage is sampled when a valid read request after CMO is recieved.
Buckets: 2x4*HPDCACHE_SETS = 8*HPDCACHE_SETS

Coverage Method: Functional Coverage

## Item 150: Cross coverage opcode (meaningfull CMO) and size

Sub Feature: Must support a CMO to invalidate a given physical address (CMO_INVAL_NLINE)

Test Case: Cross coverage opcode (meaningfull CMO) and size

Verification Goal: Purpose: Cross coverage between the opcode CMO and the size signal to cover each CMO operation. Cover the following sequence with every CMO transaction. (Write/Read(cacheable) -> CMO -> Read). The whole sequence should be on the same address.  This should be covered for each line. 
CMO_FENCE
CMO_INVAL_INLINE
CMO_INVAL_ALL
CMO_PREFETCH
Sampling Event: The coverage is sampled when a valid read request after CMO is recieved.
Buckets: 2x4*HPDCACHE_SETS = 8*HPDCACHE_SETS

Coverage Method: Functional Coverage

## Item 151: Cross coverage opcode (meaningfull CMO) and size

Sub Feature: Must support a CMO to invalidate the entire cache (CMO_INVAL_ALL)

Test Case: Cross coverage opcode (meaningfull CMO) and size

Verification Goal: Purpose: Cross coverage between the opcode CMO and the size signal to cover each CMO operation. Cover the following sequence with every CMO transaction. (Write/Read(cacheable) -> CMO -> Read). The whole sequence should be on the same address.  This should be covered for each line. 
CMO_FENCE
CMO_INVAL_INLINE
CMO_INVAL_ALL
CMO_PREFETCH
Sampling Event: The coverage is sampled when a valid read request after CMO is recieved.
Buckets: 2x4*HPDCACHE_SETS = 8*HPDCACHE_SETS

Coverage Method: Functional Coverage

## Item 152: Cross coverage opcode (meaningfull CMO) and size

Sub Feature: Must support a CMO to prefetch a given cachline (CMO_PREFETCH)

Test Case: Cross coverage opcode (meaningfull CMO) and size

Verification Goal: Purpose: Cross coverage between the opcode CMO and the size signal to cover each CMO operation. Cover the following sequence with every CMO transaction. (Write/Read(cacheable) -> CMO -> Read). The whole sequence should be on the same address.  This should be covered for each line. 
CMO_FENCE
CMO_INVAL_INLINE
CMO_INVAL_ALL
CMO_PREFETCH
Sampling Event: The coverage is sampled when a valid read request after CMO is recieved.
Buckets: 2x4*HPDCACHE_SETS = 8*HPDCACHE_SETS

Coverage Method: Functional Coverage

## Item 153 - Requirement 14.0: Check that only one memory cuts is accessed/consulted at a time (or for each request?)

Requirement ID: 14.0

Feature: Minimize number of internal RAMs activated in parallel

Feature Description: Minimize number of internal RAMs activated in parallel

Test Case: Check that only one memory cuts is accessed/consulted at a time (or for each request?)

Verification Goal: Purpose: Reduced energy consumption by limiting the number of RAMs consulted per request
Method: System verilog assertion onehot

Coverage Method: Assertion Coverage

## Item 154 - Requirement 15.0: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Requirement ID: 15.0

Feature: Enforce a non-allocate, write-through policy.

Sub Feature: All write requests from requestors must appear on the memory interface (possibly after requests are coallesced).

Feature Description: Enforce a non-allocate, write-through policy.

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? HPDCACHE_WBUF_DIR_ENTRIES: 1) +  num_rtab_entry;
        else 
            num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
        end 
        default: 
        begin
          num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES; 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 155: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? HPDCACHE_WBUF_DIR_ENTRIES : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
              default :    num_expected_write =CNT*HPDCACHE_WBUF_DIR_ENTRIES; 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 156: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of MSHR, RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be NUM_WBUF_ENTRIES.  If the value does not match the expected a UVM_ERROR is flaged. 
Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 157: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES): 1) +  num_rtab_entry;
        else 
            num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
        end 
        default: 
        begin
          num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 158: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
              default :    num_expected_write =CNT*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 159: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES).  If the value does not match the expected a UVM_ERROR is flaged.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 160: Write only with limited back pressure

Test Case: Write only with limited back pressure

Verification Goal: Purpose :  The aim of this performance test is to show the benefits of having a write buffer on the performance. 
Check the bandwidth of cached write at the core interface of HPDcache when writes arrive with no inter request delay. 
If memory response reply within a delay < NUM_WBUF cycles, the cache should be able to accept every write. 
Target     : Write Performance 
Scenario:  Write only test. 
Stimuli   :  Test is derived from random access tests.  
Requirement : 
Following condition should be respected to get full performance 
1. Store accesses are cacheable 
2. There is not inter request delay @core interface 
3. Memory response model reply within minimum (< NUM_WBUF cycles) delay possible 

Constraints: 
OP = HPDCACHE_REQ_STORE, uncacheable=0. 
The wbuf and rtab should be configured to get the maximum output. Following values are used:
                 m_cfg_wbuf_threshold = > 2 < HPDCACHE_WBUF_DIR_ENTRIES
                 m_cfg_wbuf_reset_timecnt_on_write = 1;
                 m_cfg_wbuf_sequential_waw = 0;
                 m_cfg_wbuf_inhibit_write_coalescing = 0;
                 m_cfg_rtab_single_entry = 0

All writes are done at unique SET where no request is on the fly.

Criteria Pass Fail: 
Criteria (PASS/FAIL): 
A check is performed at the end of test in report phase.
The cache is expected to accept  NB_TXN stores in NB_TXN cycles. 

In the case where write_buf_dir_entry, write_buf_data_entry < the memory write latency,  almost 3 times degrdation of performance is observed.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 161 - Requirement 16.0: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Requirement ID: 16.0

Feature: Minimize latency using a write-buffer to mask write acknowledgement delay.

Sub Feature: HPDcache uses write buffer to store a write request in writer buffer to avoid memory response delay

Feature Description: Minimize latency using a write-buffer to mask write acknowledgement delay.

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? HPDCACHE_WBUF_DIR_ENTRIES: 1) +  num_rtab_entry;
        else 
            num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
        end 
        default: 
        begin
          num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES; 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 162: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? HPDCACHE_WBUF_DIR_ENTRIES : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
              default :    num_expected_write =CNT*HPDCACHE_WBUF_DIR_ENTRIES; 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 163: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES): 1) +  num_rtab_entry;
        else 
            num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
        end 
        default: 
        begin
          num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 164: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
              default :    num_expected_write =CNT*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 165: Write only with limited back pressure

Test Case: Write only with limited back pressure

Verification Goal: Purpose :  The aim of this performance test is to show the benefits of having a write buffer on the performance. 
Check the bandwidth of cached write at the core interface of HPDcache when writes arrive with no inter request delay. 
If memory response reply within a delay < NUM_WBUF cycles, the cache should be able to accept every write. 
Target     : Write Performance 
Scenario:  Write only test. 
Stimuli   :  Test is derived from random access tests.  
Requirement : 
Following condition should be respected to get full performance 
1. Store accesses are cacheable 
2. There is not inter request delay @core interface 
3. Memory response model reply within minimum (< NUM_WBUF cycles) delay possible 

Constraints: 
OP = HPDCACHE_REQ_STORE, uncacheable=0. 
The wbuf and rtab should be configured to get the maximum output. Following values are used:
                 m_cfg_wbuf_threshold = > 2 < HPDCACHE_WBUF_DIR_ENTRIES
                 m_cfg_wbuf_reset_timecnt_on_write = 1;
                 m_cfg_wbuf_sequential_waw = 0;
                 m_cfg_wbuf_inhibit_write_coalescing = 0;
                 m_cfg_rtab_single_entry = 0

All writes are done at unique SET where no request is on the fly.

Criteria Pass Fail: 
Criteria (PASS/FAIL): 
A check is performed at the end of test in report phase.
The cache is expected to accept  NB_TXN stores in NB_TXN cycles. 

In the case where write_buf_dir_entry, write_buf_data_entry < the memory write latency,  almost 3 times degrdation of performance is observed.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 166 - Requirement 17.0: test_reg_bit_bash

Requirement ID: 17.0

Feature: Provide a memory-mapped CSR configuration interface

Sub Feature: Provide CSRs for configuration

Feature Description: Provide a memory-mapped CSR configuration interface

Test Case: test_reg_bit_bash

Verification Goal: Built-in UVM sequences to check CSRs via reg model

Criteria Pass Fail: UVM regmodel flags UVM_ERROR in case of a mismatch

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 167: test reg access sequence

Test Case: test reg access sequence

Verification Goal: Built-in UVM sequences to check CSRs via reg model

Criteria Pass Fail: UVM regmodel flags UVM_ERROR in case of a mismatch

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 168: test reg hw reset register test

Test Case: test reg hw reset register test

Verification Goal: Built-in UVM sequences to check CSRs via reg model

Criteria Pass Fail: UVM regmodel flags UVM_ERROR in case of a mismatch

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 169: cfig_version.MinorID

Test Case: cfig_version.MinorID

Verification Goal: Purpose: Cover Major Version ID of the HPDcache

Coverage Method: Functional Coverage

## Item 170: cfig_version.MajorID

Test Case: cfig_version.MajorID

Verification Goal: Purpose: Cover Minor Version ID of the HPDcache

Coverage Method: Functional Coverage

## Item 171: cfig_version.IpID

Test Case: cfig_version.IpID

Verification Goal: Purpose: Cover IP ID of the HPDcache

Coverage Method: Functional Coverage

## Item 172: cfig_version.VendorID

Test Case: cfig_version.VendorID

Verification Goal: Purpose:  Cover Vendor ID

Coverage Method: Functional Coverage

## Item 173: Number of SETs (cfig_info.Sets)

Test Case: Number of SETs (cfig_info.Sets)

Verification Goal: Purpose:  Cover Number of sets in the cache (one-based)
Method: Cover group in the test base 
Sampling Event: In the post shutdown phase 
Bucket:  3 (64, 128, 256)

Coverage Method: Functional Coverage

## Item 174: Number of Ways (cfig_info.Ways)

Test Case: Number of Ways (cfig_info.Ways)

Verification Goal: Purpose: Cover Number of ways in the cache (one-based)
Method: Cover group in the test base 
Sampling Event: In the post shutdown phase 
Bucket:  3 (2, 4, 8)

Coverage Method: Functional Coverage

## Item 175: Number of bytes per cacheline (cfig_info.CIBytes)

Test Case: Number of bytes per cacheline (cfig_info.CIBytes)

Verification Goal: Purpose: Cover Number of bytes per cacheline (power of 2)
Method: Cover group in the test base 
Sampling Event: In the post shutdown phase

Coverage Method: Functional Coverage

## Item 176: Number of sets in MSHR (cfig_info.MSHRSets)

Test Case: Number of sets in MSHR (cfig_info.MSHRSets)

Verification Goal: Purpose: Cover Number of sets in the MSHR (one-based)
Method: Cover group in the test base 
Sampling Event: In the post shutdown phase 
Buckets:  3 (1, 4, 64)

Coverage Method: Functional Coverage

## Item 177: Number of Ways in MSHR ((cfig_info.MSHRWays)

Test Case: Number of Ways in MSHR ((cfig_info.MSHRWays)

Verification Goal: Purpose: Cover Number of ways in the MSHR (one-based)
Method: Cover group in the test base 
Sampling Event: In the post shutdown phase 
Buckets:  3 (1, 2, 4,)

Coverage Method: Functional Coverage

## Item 178: Number of RTAB entries (cfig_info2.RTAB)

Test Case: Number of RTAB entries (cfig_info2.RTAB)

Verification Goal: Purpose: Cover Number of entries in the RTAB (one-based)
Method: Cover group in the test base 
Sampling Event: In the post shutdown phase 
Buckets: 3 (2, 4, 8)

Coverage Method: Functional Coverage

## Item 179: Number of Entries in the Directory of WBUF (cfig_info2.WbufDir)

Test Case: Number of Entries in the Directory of WBUF (cfig_info2.WbufDir)

Verification Goal: Purpose: Cover Number of entries in the directory of the Write Buffer (one-based)
Method: Cover group in the test base 
Sampling Event: In the post shutdown phase 
Bucket:  3 ( 4, 8, 16)

Coverage Method: Functional Coverage

## Item 180: Number of Entries in the data buffer of WBUF (cfig_info2.WbufData)

Test Case: Number of Entries in the data buffer of WBUF (cfig_info2.WbufData)

Verification Goal: Purpose: Cover Number of entries in the data buffer of the Write Buffer (one-based)
Method: Cover group in the test base 
Sampling Event: In the post shutdown phase 
Bucket: 3 (2, 4, 8)

Coverage Method: Functional Coverage

## Item 181: Nymber of bytes per Write-Buffer Data Entry (cfig_info2.WbufBytes)

Test Case: Nymber of bytes per Write-Buffer Data Entry (cfig_info2.WbufBytes)

Verification Goal: Purpose: Cover Number of bytes per Write-Buffer Data Entry (power of 2)
Method: Cover group in the test base 
Sampling Event: In the post shutdown phase 
Bucket:  3 (2, 4, 8)

Coverage Method: Functional Coverage

## Item 182: Cache enable  ( cfig_cachectrl.E )

Test Case: Cache enable  ( cfig_cachectrl.E )

Verification Goal: Purpose: Cache Enable - When set to 0, all memory accesses are considered uncacheable
Method: Cover group in the test base 
Sample Event: this coverage is sampled at the end of the test in report phase once the counters are read and verified. 
Buckets: 2

Coverage Method: Functional Coverage

## Item 183: Performance Counter  ( cfig_cachectrl.P )

Test Case: Performance Counter  ( cfig_cachectrl.P )

Verification Goal: Purpose: This bit indicates if performance counters are enabled. Coverage that all values of this signal have been covered. 
Method: Cover group in the test base 
Sample Event: This coverage is sampled at the end of the test in report phase once the counters are read and verified. 
Buckets: 1 bit => 2 buckets

Coverage Method: Functional Coverage

## Item 184: Rtab single entry (cfig_cachectrl.R)

Test Case: Rtab single entry (cfig_cachectrl.R)

Verification Goal: Purpose: Single-Entry RTAB (fallback mode) - When set to 1, the cache controller only uses one entry of the Replay Table.
Sample Event: This coverage is sampled at the end of congeston tests described in section 1.3. 
Buckets: 1 bit => 2 buckets

Coverage Method: Functional Coverage

## Item 185: Wbuf threshold (cfig_wbuf.T)

Test Case: Wbuf threshold (cfig_wbuf.T)

Verification Goal: Purpose: Time-counter Threshold - This field defines the time-counter threshold on which open write-buffer entries (OPEN) go to the pending state (PEND)
Method: Cover group in the test base 
Sample Event: This coverage is sampled at the end of congeston tests described in section 1.3. 
Buckets: ??

Coverage Method: Functional Coverage

## Item 186: Wbuf reset timecnt on write (cfig_wbuf.R)

Test Case: Wbuf reset timecnt on write (cfig_wbuf.R)

Verification Goal: Purpose: Reset time-counter on write - When set to 1, write accesses restart the time-counter to 0 of the used write-buffer entry
Method: Cover group in the test base 
Sample Event: This coverage is sampled at the end of congeston tests described in section 1.3. 
Buckets: 1 bit => 2 buckets

Coverage Method: Functional Coverage

## Item 187: Wbuf sequential Write-After-Write (cfig_wbuf.S)

Test Case: Wbuf sequential Write-After-Write (cfig_wbuf.S)

Verification Goal: Purpose: Sequential Write after Write - When set to 1, the write buffer stalls write accesses that collide with an in-flight write transaction (SENT).
Method: Cover group in the test base 
Sample Event: This coverage is sampled at the end of congeston tests described in section 1.3. 
Buckets: 1 bit => 2 buckets

Coverage Method: Functional Coverage

## Item 188: Inhibit Write Buf Coalescing (cfig_wbuf.I)

Test Case: Inhibit Write Buf Coalescing (cfig_wbuf.I)

Verification Goal: Purpose: Inhibit Write Coalescing - When set to 1, entries in the write-buffer go from the FREE state to the PEND state directly (bypassing the OPEN state). Moreover, no coalescing is accepted while the entry is in the PEND state.
Method: Cover group in the test base 
Sample Event: This coverage is sampled at the end of congeston tests described in section 1.3. 
Buckets: 1 bit => 2 buckets

Coverage Method: Functional Coverage

## Item 189: Performance counter check

Sub Feature: Provide CSRs for performance monitoring

Test Case: Performance counter check

Verification Goal: Purpose: Check to verify if the performance counters are correct. 
Method: This check is performed in report phase, once the test is finished. The counters are predicted in the SB and are compared against the register read value.

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 190: test_reg_bit_bash

Test Case: test_reg_bit_bash

Verification Goal: Built-in UVM sequences to check CSRs via reg model

Criteria Pass Fail: UVM regmodel flags UVM_ERROR in case of a mismatch

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 191: test reg access sequence

Test Case: test reg access sequence

Verification Goal: Built-in UVM sequences to check CSRs via reg model

Criteria Pass Fail: UVM regmodel flags UVM_ERROR in case of a mismatch

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 192: test reg hw reset register test

Test Case: test reg hw reset register test

Verification Goal: Built-in UVM sequences to check CSRs via reg model

Criteria Pass Fail: UVM regmodel flags UVM_ERROR in case of a mismatch

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 193: Performance Counter  ( cfig_cachectrl.P )

Test Case: Performance Counter  ( cfig_cachectrl.P )

Verification Goal: Purpose: This bit indicates if performance counters are enabled. Coverage that all values of this signal have been covered. 
Method: Cover group in the test base 
Sample Event: This coverage is sampled at the end of the test in report phase once the counters are read and verified. 
Buckets: 1 bit => 2 buckets

Coverage Method: Functional Coverage

## Item 194: perf_write_cnt : 64-bit counter for processed write requests

Test Case: perf_write_cnt : 64-bit counter for processed write requests

Verification Goal: Purpose: Cover that each bit of counter has 0/1. Cover min (0) and Max (64hFFFFFFFFFFFFFFFF) values. 
It is impossible to reach max value in simulation, so it may require that counters are initialised (using backdoor/force signal) to some initial value which are closed to max value. Cover only if sampled value is different from initial value. 
Method: Cover group in the test base 
Sample Event: This coverage is sampled in the report phase of test_base. 
Bucket:  2+2*64 = 130

Coverage Method: Functional Coverage

## Item 195: perf_write_cnt : 64-bit counter for processed write requests

Test Case: perf_write_cnt : 64-bit counter for processed write requests

Verification Goal: Purpose: Cover that each bit of counter has 0/1. Cover min (0) and Max (64hFFFFFFFFFFFFFFFF) values. 
It is impossible to reach max value in simulation, so it may require that counters are initialised (using backdoor/force signal) to some initial value which are closed to max value. Cover only if sampled value is different from initial value. 
Method: Cover group in the test base 
Sample Event: This coverage is sampled in the report phase of test_base. 
Bucket:  2+2*64 = 130

Coverage Method: Functional Coverage

## Item 196: perf_read_cnt  : 64-bit counter for processed read requests

Test Case: perf_read_cnt  : 64-bit counter for processed read requests

Verification Goal: same as 8.2.21

Coverage Method: Functional Coverage

## Item 197: perf_prefetch_cnt   : 64-bit counter for processed prefetch requests

Test Case: perf_prefetch_cnt   : 64-bit counter for processed prefetch requests

Verification Goal: same as 8.2.21

Coverage Method: Functional Coverage

## Item 198: perf_uncached_cnt : 64-bit counter for processed uncached requests

Test Case: perf_uncached_cnt : 64-bit counter for processed uncached requests

Verification Goal: same as 8.2.21

Coverage Method: Functional Coverage

## Item 199: perf_cmo_cnt            : 64-bit counter for processed CMO requests

Test Case: perf_cmo_cnt            : 64-bit counter for processed CMO requests

Verification Goal: same as 8.2.21

Coverage Method: Functional Coverage

## Item 200: perf_accepted_cnt    : 64-bit counter for processed requests

Test Case: perf_accepted_cnt    : 64-bit counter for processed requests

Verification Goal: same as 8.2.21

Coverage Method: Functional Coverage

## Item 201: perf_write_miss_cnt : 64-bit counter for write cache misses

Test Case: perf_write_miss_cnt : 64-bit counter for write cache misses

Verification Goal: same as 8.2.21

Coverage Method: Functional Coverage

## Item 202: perf_read_miss_cnt   : 64-bit counter for read cache misses

Test Case: perf_read_miss_cnt   : 64-bit counter for read cache misses

Verification Goal: same as 8.2.21

Coverage Method: Functional Coverage

## Item 203: perf_onhold_cnt : 64-bit counter for requests put on-hold

Test Case: perf_onhold_cnt : 64-bit counter for requests put on-hold

Verification Goal: same as 8.2.21

Coverage Method: Functional Coverage

## Item 204: perf_onhold_mshr_cnt : 64-bit counter for requests put on-hold because of MSHR conflicts

Test Case: perf_onhold_mshr_cnt : 64-bit counter for requests put on-hold because of MSHR conflicts

Verification Goal: same as 8.2.21

Coverage Method: Functional Coverage

## Item 205: perf_onhold_wbuf_cnt : 64-bit counter for requests put on-hold because of WBUF conflicts

Test Case: perf_onhold_wbuf_cnt : 64-bit counter for requests put on-hold because of WBUF conflicts

Verification Goal: same as 8.2.21

Coverage Method: Functional Coverage

## Item 206: perf_onhold_rollback_cnt : 64-bit counter for requests put on-hold (again) after a rollback

Test Case: perf_onhold_rollback_cnt : 64-bit counter for requests put on-hold (again) after a rollback

Verification Goal: same as 8.2.21

Coverage Method: Functional Coverage

## Item 207: perf_stall_cnt : 64-bit counter for stall cycles (cache does not accept a request)

Test Case: perf_stall_cnt : 64-bit counter for stall cycles (cache does not accept a request)

Verification Goal: same as 8.2.21

Coverage Method: Functional Coverage

## Item 208: Features not part of test plan

Sub Feature: Provide CSRs for status monitoring

Test Case: Features not part of test plan

## Item 209: CSR Verification

Sub Feature: The CSR space must be shared between requesters, by inaccessible from the memory interface.

Test Case: CSR Verification

Verification Goal: Purpose: To check the value of the CSRs. Some of the CSRs are systematically read in the post shutdown phase, like performance counters, cfg_info, status registers. 
Method: UVM_REG model is used to read and write status and configuration registers. 
Randomization: Some of the counter signals are initialized (by backdoor forcing) to some very high random values to be able to achieve overflow.

Test Type: Directed Self-Checking

Coverage Method: Assertion Coverage

## Item 210 - Requirement 18.0: Features not part of test plan

Requirement ID: 18.0

Feature: Provide a memory interface wrapper which is compatible with AXI5.

Sub Feature: When used with this wrapper, the memory interface must be compliant with AXI5.

Feature Description: Provide a memory interface wrapper which is compatible with AXI5.

Test Case: Features not part of test plan

## Item 211: Features not part of test plan

Sub Feature: When used with this wrapper, all functions of the cache must operate correctly, and the same as without the wrapper.

Test Case: Features not part of test plan

## Item 212 - Requirement 19.0: Parameter Coverage

Requirement ID: 19.0

Feature: Provide RTL whose functionality is configurable based on numerous System Verilog parameters

Sub Feature: The design that is provided must work correctly when compiled with any combination of parameters given in the tab "CONFIGURATIONS"

Feature Description: Provide RTL whose functionality is configurable based on numerous System Verilog parameters

Test Case: Parameter Coverage

Verification Goal: The value covered for each parameters are listed as value for HPC1, value for HPC2, value for EMBEDDED1, value for EMBEDDED2.

## Item 213: hpdcache_PA_WIDTH

Sub Feature: Provide a configurable physical addres width via CONF_HPDCACHE_PA_WIDTH

Test Case: hpdcache_PA_WIDTH

Verification Goal: Physical address width (in bits) : 49, 49, 32, 32

Coverage Method: Functional Coverage

## Item 214: hpdcache_WORD_WIDTH

Sub Feature: Provide a configurable data word width with via CONF_HPDCACHE_WORD_WIDTH

Test Case: hpdcache_WORD_WIDTH

Verification Goal: Width (in bits) of a data word: 64, 64, 32, 32

Coverage Method: Functional Coverage

## Item 215: hpdcache_REQ_WORDS

Sub Feature: Provide a configurable number of words in the data channels to/from requestos via CONF_HPDCACHE_REQ_WORDS

Test Case: hpdcache_REQ_WORDS

Verification Goal: Number of words in the data channels from/to requesters: 2, 1, 1, 1

Coverage Method: Functional Coverage

## Item 216: hpdcache_REQ_TRANS_ID_WIDTH

Sub Feature: Provide a configurable width for the transacion ID via CONF_HPDCACHE_REQ_TRANS_ID_WIDTH

Test Case: hpdcache_REQ_TRANS_ID_WIDTH

Verification Goal: Width (in bits) of the transaction ID from requesters: 7, 4, 3, 2

Coverage Method: Functional Coverage

## Item 217: hpdcache_REQ_SRC_ID_WIDTH

Sub Feature: Provide a configurable width for the source ID via CONF_HPDCACHE_REQ_SRC_ID_WIDTH

Test Case: hpdcache_REQ_SRC_ID_WIDTH

Verification Goal: Width (in bits) of the source ID from requesters: 3, 2, 2, 2

Coverage Method: Functional Coverage

## Item 218: hpdcache_SETS

Sub Feature: Provide a configurable number of SETs via CONF_HPDCACHE_SETS

Test Case: hpdcache_SETS

Verification Goal: Number of sets: 256, 64, 64, 128

Coverage Method: Functional Coverage

## Item 219: hpdcache_WAYS

Sub Feature: Provide a configurable number of WAYs via CONF_HPDCACHE_WAYS

Test Case: hpdcache_WAYS

Verification Goal: Number of ways (associativity): 4, 8, 2, 4

Coverage Method: Functional Coverage

## Item 220: hpdcache_CL_WORDS

Sub Feature: Provide a configurable number of words in a cacheline via CONF_HPDCACHE_CL_WORDS

Test Case: hpdcache_CL_WORDS

Verification Goal: Number of words in a cacheline: 8, 2, 4, 2

Coverage Method: Functional Coverage

## Item 221 - Requirement 19.0: hpdcache_WBUF_DIR_ENTRIES

Requirement ID: 19.0

Feature: Provide RTL whose functionality is configurable based on numerous System Verilog parameters

Sub Feature: The design that is provided must work correctly when compiled with any combination of parameters given in the tab "CONFIGURATIONS"

Feature Description: Provide RTL whose functionality is configurable based on numerous System Verilog parameters

Test Case: hpdcache_WBUF_DIR_ENTRIES

Verification Goal: Number of entries in the directory of the write buffer: 16, 8, 4, 4

Coverage Method: Functional Coverage

## Item 222: hpdcache_WBUF_DATA_ENTRIES

Sub Feature: Provide a configurable number of entries in the data portion of the write buffer via CONF_HPDCACHE_WBUF_DATA_ENTRIES

Test Case: hpdcache_WBUF_DATA_ENTRIES

Verification Goal: Number of entries in the data part of the write buffer: 8, 4, 4, 2

Coverage Method: Functional Coverage

## Item 223: hpdcache_WBUF_WORDS

Sub Feature: Provide a configurable number of words per entry in the write buffer via CONF_HPDCACHE_WBUF_WORDS

Test Case: hpdcache_WBUF_WORDS

Verification Goal: Number of data words per entry in the write buffer: 4, 1, 2, 1

Coverage Method: Functional Coverage

## Item 224: hpdcache_WBUF_TIMECNT_MAX

Sub Feature: Provide a configurable width for the time counter in the write buffers via CONF_HPDCACHE_WBUF_TIMECNT_MAX

Test Case: hpdcache_WBUF_TIMECNT_MAX

Verification Goal: Maximumvalue of the time counter in write buffer entries: 7, 5, 5, 3

Coverage Method: Functional Coverage

## Item 225: hpdcache_RTAB_ENTRIES

Sub Feature: Provide a configurable number of entries in the replay table via CONF_HPDCACHE_RTAB_ENTRIES

Test Case: hpdcache_RTAB_ENTRIES

Verification Goal: Number of entries in the replay table: 8, 4, 4, 2

Coverage Method: Functional Coverage

## Item 226: hpdcache_MSHR_SETS

Sub Feature: Provide a configurable number of sets in the MSHR via CONF_HPDCACHE_MSHR_SETS

Test Case: hpdcache_MSHR_SETS

Verification Goal: Number of sets in theMiss Status Holding Register (MSHR): 64, 4, 1, 1

Coverage Method: Functional Coverage

## Item 227: hpdcache_MSHR_WAYS

Sub Feature: Provide a configurable number of ways in the MSHR via CONF_HPDCACHE_MSHR_WAYS

Test Case: hpdcache_MSHR_WAYS

Verification Goal: Number of ways (associativity) in the MSHR: 2, 4, 1, 4

Coverage Method: Functional Coverage

## Item 228: hpdcache_MEM_ID_WIDTH

Sub Feature: hpdcache_MEM_ID_WIDTH

Test Case: hpdcache_MEM_ID_WIDTH

Verification Goal: Width (in bits) of the memory transaction ID: 8, 5, 4, 3

Coverage Method: Functional Coverage

## Item 229: ID coverage

Test Case: ID coverage

Verification Goal: Purpose: Coverage that each value of this signal was covered
Buckets: HPDCACHE_MEM_ID_WIDTH bits => 2**HPDCACHE_MEM_ID_WIDTH buckets
Samlping Event: Sample at the time of NoC Reponse

Coverage Method: Functional Coverage

## Item 230: W_id coverage

Test Case: W_id coverage

Verification Goal: Purpose: Coverage that each value of this signal was covered
Buckets: HPDCACHE_MEM_ID_WIDTH bits => 2**HPDCACHE_MEM_ID_WIDTH buckets
Sampling Event: Sample at the time of NoC Reponse . Sample when the NoC response is witout an error.

Coverage Method: Functional Coverage

## Item 231: hpdcache_MEM_DATA_WIDTH

Sub Feature: hpdcache_MEM_DATA_WIDTH

Test Case: hpdcache_MEM_DATA_WIDTH

Verification Goal: Width (in bits) of the memory data: 512, 64, 128, 32

Coverage Method: Functional Coverage

## Item 232: hpdcache_MSHR_WAYS_PER_RAM_WORD

Sub Feature: Control the number of ways in the same MSHR SRAM word via CONF_HPDCACHE_MSHR_WAYS_PER_RAM_WORD

Test Case: hpdcache_MSHR_WAYS_PER_RAM_WORD

Verification Goal: Number of ways in the same MSHR SRAM word: 2, 2, 1, 2

Coverage Method: Functional Coverage

## Item 233: hpdcache_MSHR_SETS_PER_RAM

Sub Feature: Provide a configurable physical addres width via CONF_HPDCACHE_PA_WIDTH

Test Case: hpdcache_MSHR_SETS_PER_RAM

Verification Goal: Number of sets per RAM macro in the MSHR array of the cache: 64, 4, 1, 1

Coverage Method: Functional Coverage

## Item 234: hpdcache_DATA_WAYS_PER_RAM_WORD

Sub Feature: Control the number of ways in the same CACHE data SRAM word via CONF_HPDCACHE_DATA_WAYS_PER_RAM_WORD

Test Case: hpdcache_DATA_WAYS_PER_RAM_WORD

Verification Goal: Number of ways in the same CACHE data SRAM word: 2, 2, 2, 4

Coverage Method: Functional Coverage

## Item 235: hpdcache_DATA_SETS_PER_RAM

Sub Feature: Control the number of sets per RAM macro in the data array via CONF_HPDCACHE_DATA_SETS_PER_RAM

Test Case: hpdcache_DATA_SETS_PER_RAM

Verification Goal: Number of sets per RAM macro in the DATA array of the cache: 128, 32, 64, 64

Coverage Method: Functional Coverage

## Item 236: hpdcache_ACCESS_WORDS

Sub Feature: Control the number of words of a given set that can be accessed simultaneously via CONF_HPDCACHE_ACCESS_WORDS

Test Case: hpdcache_ACCESS_WORDS

Verification Goal: Number of words of a given SET that can be accessed simultaneously from the CACHE data array: 4, 2, 2, 1

Coverage Method: Functional Coverage

## Item 237: hpdcache_VICTIM_SEL

Sub Feature: Configure victim_sel via CONF_HPDCACHE_VICTIM_SEL

Test Case: hpdcache_VICTIM_SEL

Verification Goal: Enable or disable victim_sel feature: 1, 0, 0, 1

## Item 238 - Requirement 20.0: Parameter Coverage

Requirement ID: 20.0

Feature: Provide RTL such that the partitioning of data into SRAM cuts is configurable

Sub Feature: Regardless of the partitioning of the SRAM cuts, the cache functionality must be correct.

Feature Description: Provide RTL such that the partitioning of data into SRAM cuts is configurable

Test Case: Parameter Coverage

Verification Goal: The value covered for each parameters are listed as value for HPC1, value for HPC2, value for EMBEDDED1, value for EMBEDDED2.

## Item 239: hpdcache_MSHR_WAYS_PER_RAM_WORD

Sub Feature: Control the number of ways in the same MSHR SRAM word via CONF_HPDCACHE_MSHR_WAYS_PER_RAM_WORD

Test Case: hpdcache_MSHR_WAYS_PER_RAM_WORD

Verification Goal: Number of ways in the same MSHR SRAM word: 2, 2, 1, 2

Coverage Method: Functional Coverage

## Item 240: hpdcache_MSHR_SETS_PER_RAM

Sub Feature: Control the number of sets per RAM macro in the MSHR array via CONF_HPDCACHE_MSHR_SETS_PER_RAM

Test Case: hpdcache_MSHR_SETS_PER_RAM

Verification Goal: Number of sets per RAM macro in the MSHR array of the cache: 64, 4, 1, 1

Coverage Method: Functional Coverage

## Item 241: hpdcache_DATA_WAYS_PER_RAM_WORD

Sub Feature: Control the number of ways in the same CACHE data SRAM word via CONF_HPDCACHE_DATA_WAYS_PER_RAM_WORD

Test Case: hpdcache_DATA_WAYS_PER_RAM_WORD

Verification Goal: Number of ways in the same CACHE data SRAM word: 2, 2, 2, 4

Coverage Method: Functional Coverage

## Item 242: hpdcache_DATA_SETS_PER_RAM

Sub Feature: Control the number of sets per RAM macro in the data array via CONF_HPDCACHE_DATA_SETS_PER_RAM

Test Case: hpdcache_DATA_SETS_PER_RAM

Verification Goal: Number of sets per RAM macro in the DATA array of the cache: 128, 32, 64, 64

Coverage Method: Functional Coverage

## Item 243: hpdcache_ACCESS_WORDS

Sub Feature: Control the number of words of a given set that can be accessed simultaneously via CONF_HPDCACHE_ACCESS_WORDS

Test Case: hpdcache_ACCESS_WORDS

Verification Goal: Number of words of a given SET that can be accessed simultaneously from the CACHE data array: 4, 2, 2, 1

Coverage Method: Functional Coverage

## Item 244 - Requirement 22.0: Reset On The Fly

Requirement ID: 22.0

Feature: Provide an asynchronous, active low reset

Sub Feature: Single external resetpin RST_NI. After application of the reset, the cache must restore the default directory and CSR state

Feature Description: Provide an asynchronous, active low reset

Test Case: Reset On The Fly

Verification Goal: Purpose:  To apply reset on the fly randomly. 
Method:  There is a configuration variable, m_reset_on_the_fly on the hpdcache_top_config. When this variable is enabled reset is applied. The reset uses jump mechanism of the UVM (jump to reset phase from main phase). The reusable reset agent is used to apply jump. 
Randomization:     Following formula is used to apply the reset. Reset is applied either when req_count is somewhere between 40 and 1000 or clk_cnt == 10000. 
     if((env.m_hpdcache_sb.get_req_counter() == $urandom_range(4000, 5000)) || (clk_cnt == 10000)) env.reset_driver.emit_assert_reset();
Link:  Reusable UVM agent reset driver of cv_dv_utils  is used to apply the reset.
Expected Behaviour: 
We expect the HPDcache to reinitialize all internal buffers (FIFOs, RTAB, WBUF, MSHR, Dcache Directory).

Test Type: ENV capability, not specific test

Coverage Method: Code Coverage

## Item 245: SHCMOO 1 reset   @hpdcache core request

Test Case: SHCMOO 1 reset   @hpdcache core request

Verification Goal: Purpose: The purpose of this assertion is to make sure that hpdcache works correctly when a reset is asserted around a request on the core interface. 
Description: If reset arrives at time T and valid is inserted at T1. Cover that T1 arrives between [T-T1:T+T1] (T1 = 5)
This assertion covers the cases where ready is 1 or 0 when the valid is 1. 
This assertion covers the cases where request arrives with phy_index = 0 or 1. 
Trigger and clock event: The clock is the clk hpdcache. The assertion is triggered by a rising edge on reset or req valid.
Number of Cases: 11x2(ready or not)x2(phys_indexed or not) == 44
Link: hpdcache_sva.sv

Coverage Method: Assertion Coverage

## Item 246: SHCMOO 2 reset  @hpdcache core response

Test Case: SHCMOO 2 reset  @hpdcache core response

Verification Goal: Purpose: The purpose of this assertion is to make sure that hpdcache works correctly when a reset is asserted around a reponse on the core interface. 
Description: If reset arrives at time T and valid is inserted at T1. Cover that T1 arrives between [T-T1:T+T1] (T1 = 5)
This assertion covers the cases where reset is asserted when  a response arrives with or without a error response. 
This assertion covers the cases where reset is asserted when a  response arrives with or without a abort response. 
Trigger and clock event: The clock is the clk hpdcache. The assertion is triggered by a rising edge on reset or rsp valid.
Number of Cases: 11x2 + 11x2(abort or not) == 44
Link: hpdcache_sva.sv

Coverage Method: Assertion Coverage

## Item 247: SHCMOO 3 reset @memory read  request

Test Case: SHCMOO 3 reset @memory read  request

Verification Goal: Same as 4.1 , in this case reset is asserted when the request arrives at memory read interface. 
This assertion covers the cases where reset arrives when a request is cached or uncached. 
Number of Cases: 11x2 == 22
Link: hpdcache_sva.sv

Coverage Method: Assertion Coverage

## Item 248: SHCMOO 4 reset @memory write request

Test Case: SHCMOO 4 reset @memory write request

Verification Goal: same as 4.3, in this case reset is asserted when the request arrives at memory write interface
This assertion also cover the case where reset arrives when a request is a AMO.

Coverage Method: Assertion Coverage

## Item 249: SHCMOO 5 reset @memory read response

Test Case: SHCMOO 5 reset @memory read response

Verification Goal: same as 4.3, in this case reset is asserted when  a response arrives at memory read interface
This assertion also covers the cases where reset arrives when a response arrives with or without an error.
This assertion also covers the cases where reset arrives when a response arrives with or without ex fail .
Number of Cases: 11x2(error or not) + 11x2(ex fail or not) == 44
Link: hpdcache_sva.sv

Coverage Method: Assertion Coverage

## Item 250: SHCMOO 6 reset @memory write response

Test Case: SHCMOO 6 reset @memory write response

Verification Goal: same as 4.5, in this case reset is asserted when the a response arrives at memory write interface

Coverage Method: Assertion Coverage

## Item 251: SCHMOO 7 reset @ flush

Test Case: SCHMOO 7 reset @ flush

Verification Goal: Purpose: The prupose of this assertion is to make sure that hpdcache works correctly when a reset is asserted when flush is going on. 
Description: If reset arrives at time T and flush is inserted at T1. Cover that T1 arrives between [T-T1:T+T1] (T1 = 5)
This assertion should be covered when many cached write are on the fly. 
Link: hpdcache_sva.sv

Coverage Method: Assertion Coverage

## Item 252: Back to Back to same addresse interleaved with reset

Test Case: Back to Back to same addresse interleaved with reset

Verification Goal: Purpose: The purpose of thisassertion is to make sure that a new request after a reset is always miss. 
Desciption: If a request is sent @ A followed by a reset follow by another request @A, we expect that the new resuest after the reset would be a miss. 
Link: hpdcache_sva.sv

Coverage Method: Assertion Coverage

## Item 253: No request after the reset

Test Case: No request after the reset

Verification Goal: Purpose: The purpose of this assertion is to make sure that HPDcache does not send spurious memory request after a reset is asserted
Desciption: Wait for 10 cycles after the reset before sending a new core request. During this 10 cycles HPDCache shall not make new memory request. It is a coverage as well as a check. 
Link: hpdcache_sva.sv

Coverage Method: Assertion Coverage

## Item 254: test reg hw reset register test

Test Case: test reg hw reset register test

Verification Goal: Built-in UVM sequences to check CSRs via reg model

Criteria Pass Fail: UVM regmodel flags UVM_ERROR in case of a mismatch

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 255: Arrays / fifos empty

Test Case: Arrays / fifos empty

Criteria Pass Fail: Criteria (PASS/FAILED): If an array or fifo is not empty the asserion fails and an error is flaged.

Coverage Method: Assertion Coverage

## Item 256: RTAB empty

Test Case: RTAB empty

Verification Goal: Purpose  : To determine if rtab is empty after at the end of the test
Method   : Assertions are used on the internal signal

Trigger Event: Post shutdown phase,  under reset

Criteria Pass Fail: Criteria (PASS/FAILED): If rtab is not empty the asserion fails and an error is flaged.

Coverage Method: Assertion Coverage

## Item 257: Wbuf empty

Test Case: Wbuf empty

Verification Goal: Purpose  : To determine if write buffer is empty after at the end of the test
Method   : Assertions are used on the internal signal

Trigger Event: Post shutdown phase,  under reset

Criteria Pass Fail: Criteria (PASS/FAILED): If write buffer is not empty the asserion fails and an error is flaged.

Coverage Method: Assertion Coverage

## Item 258: FSM state end of test

Test Case: FSM state end of test

Verification Goal: Purpose  : To determine that the internal FSM are in their end of test state (wbuf/refill/miss req/cmo)
Method   : Assertions are used on the internal signal

Trigger Event: Post shutdown phase ,  under reset

Criteria Pass Fail: Criteria (PASS/FAILED): If internal FSM are not in their end of test state the asserion fails and an error is flaged.

Coverage Method: Assertion Coverage

## Item 259: Directory Coherency

Test Case: Directory Coherency

Verification Goal: Purpose  : Check at the end of the test that the directory is coherent with tag directory
Method   : The SV based model of directory is used. The SV based model is directly  bind to the mem controller module of HPDcache and uses internal signal to get the directory state. 
RTL directory is read via backdoor. 

Trigger Event: Post shutdown phase

Criteria Pass Fail: Criteria (PASS/FAILED):  If there is mismatch a UVM_ERROR is flaged

Coverage Method: Assertion Coverage

## Item 260: Memory coherency (External Memory and Shadow Memory)

Test Case: Memory coherency (External Memory and Shadow Memory)

Verification Goal: Purpose: Check at the end of the test that the data in the external memory is coherent with the data in the shadow external memory
Method: The UVM based model is compared with the memory within memory response model

Trigger Event: Post shutdown phase

Criteria Pass Fail: Criteria : If there is a mimsatch a UVM_ERROR is flaged

Coverage Method: Assertion Coverage

## Item 261: Counter Checks

Test Case: Counter Checks

Verification Goal: Purpose: Predict the hpdcache performance counter vaules 

The hpdcache has following counters. These counters are predicted in the TB of HPDcache. 
At the end of the test in the report phase, the value of these counters are read (from registers) and compared against the reference counters. 
These counters may require to be initialised by backdoor to be able to achieve all values. 

perf_write_cnt : 64-bit counter for processed write requests
perf_read_cnt  : 64-bit counter for processed read requests
perf_prefetch_cnt   : 64-bit counter for processed prefetch requests
perf_uncached_cnt : 64-bit counter for processed uncached requests
perf_cmo_cnt          : 64-bit counter for processed CMO requests
perf_accepted_cnt    : 64-bit counter for processed requests
perf_write_miss_cnt : 64-bit counter for write cache misses
perf_read_miss_cnt   : 64-bit counter for read cache misses
perf_onhold_cnt : 64-bit counter for requests put on-hold
perf_onhold_mshr_cnt : 64-bit counter for requests put on-hold because of MSHR conflicts (how to predict ?)
perf_onhold_wbuf_cnt : 64-bit counter for requests put on-hold because of WBUF conflicts(how to predict ?)
perf_onhold_rollback_cnt : 64-bit counter for requests put on-hold (again) after a rollback(how to predict ?)
perf_stall_cnt : 64-bit counter for stall cycles (cache does not accept a request)


Method:  In the post shutdown phase, counter registers are read and compared with the value predicted within the UVM TB. 

Trigger Event: Post shutdown phase

Criteria Pass Fail: Criteria : If there is a mimsatch a UVM_ERROR is flaged. In the cases where it is difficult to predict exact values, fuzzy values are used (the counters to be precised)

Coverage Method: Assertion Coverage

## Item 262 - Requirement 23.0: Flush on the fly

Requirement ID: 23.0

Feature: Provide the ability to initiate a flush of the write buffer.

Sub Feature: Initiate a flush of the write buffer via a one-cycle pulse on the input pin WBUF_FLUSH_I

Feature Description: Provide the ability to initiate a flush of the write buffer.

Test Case: Flush on the fly

Verification Goal: Purpose:  To apply flush on the fly randomly. 
Method:  There is a configuration variable, m_flush_on_the_fly on the hpdcache_top_config. When this variable is enabled flush is applied. The reusable pulse agent is used to generate pulses. It is configured to apply 10 pulses of 1 cycle each, every 2000 cycles.
Randomization:    Following configuration is used to apply the clock based synchronous pulses. 
    env.m_flush_cfg.set_pulse_enable(m_flush_on_the_fly);
    env.m_flush_cfg.set_pulse_clock_based(1);
    env.m_flush_cfg.set_pulse_width(1);
    env.m_flush_cfg.set_pulse_period(5000);
    env.m_flush_cfg.set_pulse_phase_shift(0);
    env.m_flush_cfg.set_pulse_num(5);

Link:  Reusable UVM agent pulse_gen of cv_dv_utils is used to generate the flush
Expected Behavior: 
After a flush, the HPDcache shall not accept new write requests (all core_req_ready_o are reset to 0) until wbuf_empty_o is asserted

Test Type: ENV capability, not specific test

Coverage Method: Code Coverage

## Item 263: SHCMOO 1 flush   @hpdcache core request

Test Case: SHCMOO 1 flush   @hpdcache core request

Verification Goal: Purpose: The prupose of this assertion is to make sure that hpdcache works correctly when a flush is asserted around a request on the core interface. 
Description: If flush arrives at time T and valid is inserted at T1. Cover that T1 arrives between [T-T1:T+T1] (T1 = 5)
This assertion is covers the cases where ready is 1 or 0 when the valid is 1. 
This assertion covers the cases where request arrives with phy_index = 0 or 1. 
Trigger and clock event: The clock is the clk hpdcache. The assertion is triggered by a rising edge on flush or req valid.
Number of Cases: 11x2(ready or not)x2(phys_indexed or not) == 44
Link: hpdcache_sva.sv

Coverage Method: Assertion Coverage

## Item 264: SHCMOO 2 flush @hpdcache core response

Test Case: SHCMOO 2 flush @hpdcache core response

Verification Goal: Purpose: The prupose of this assertion is to make sure that hpdcache works correctly when a flush is asserted around a reponse on the core interface. 
Descirption: If flush arrives at time T and valid is inserted at T1. Cover that T1 arrives between [T-T1:T+T1] (T1 = 5)
This assertion covers the cases where flush is asserted when  a response arrives with or without a error response. 
This assertion covers the cases where flush is asserted when a  response arrives with or without a abort response. 
Number of Cases: 11x2 + 11x2(abort or not) == 44
Link: hpdcache_sva.sv

Coverage Method: Assertion Coverage

## Item 265: SHCMOO 3 flush @memory read  request

Test Case: SHCMOO 3 flush @memory read  request

Verification Goal: Same as 5.1 , in this case flush is asserted when the request arrives at memory read interface. 
This assertion covers the cases where flush arrives when a request is cached or uncached. 
Number of Cases: 11x2 == 22
Link: hpdcache_sva.sv

Coverage Method: Assertion Coverage

## Item 266: SHCMOO 4 flush @memory write request

Test Case: SHCMOO 4 flush @memory write request

Verification Goal: same as 5.3, in this case flush is asserted when the request arrives at memory write interface
This assertion also cover the case where flush arrives when a request is a AMO.

Coverage Method: Assertion Coverage

## Item 267: SHCMOO 5 flush @memory read response

Test Case: SHCMOO 5 flush @memory read response

Verification Goal: same as 5.3, in this case flush is asserted when  a response arrives at memory read interface
This assertion also covers the cases where flush arrives when a response arrives with or without an error.
This assertion also covers the cases where flush arrives when a response arrives with or without ex fail  
Number of Cases: 11x2 + 11x2 == 44
Link: hpdcache_sva.sv

Coverage Method: Assertion Coverage

## Item 268: SHCMOO 6 flush @memory write response

Test Case: SHCMOO 6 flush @memory write response

Verification Goal: same as 5.5, in this case flush is asserted when the a response arrives at memory write interface

Coverage Method: Assertion Coverage

## Item 269 - Requirement 24.0: Ready Valid Protocol

Requirement ID: 24.0

Feature: Respect the ready/valid protocol on all requester and memory interfaces

Sub Feature: Once VALID is set to '1' it must stay at '1' until RDY goes to '1' and the transfer occurs.

Feature Description: Respect the ready/valid protocol on all requester and memory interfaces

Test Case: Ready Valid Protocol

Verification Goal: Purpose: Cover following scenerios of ready valid protocol: 
1. Read and Valid are inserted at the same time 
2. Ready is high before the valid is inserted 
3. Valid is high before the ready in inserte
Sampling Event: When a valid request is recieved. 
Buckets: 3 buckets

Coverage Method: Assertion Coverage

## Item 270 - Requirement 25.0: Schmoo 1 (hpdcache req  -> memory req interface)

Requirement ID: 25.0

Feature: The cache must be able to put certain requests on-hold in the RTAB and re-execute them later, while still assuring the Memory Consistency Rules (MCRs).

Sub Feature: In case 1 (cacheable Load or pre-fetch) which hits on a pending miss - the 2nd read must wait for 1st to complete to get data.

Feature Description: The cache must be able to put certain requests on-hold in the RTAB and re-execute them later, while still assuring the Memory Consistency Rules (MCRs).

Test Case: Schmoo 1 (hpdcache req  -> memory req interface)

Verification Goal: Purpose: This schmoo is intended to cover the temporal relationship between a processor operation for a cache line and that cache line address appearing on a AXI request  transaction( from a previous request on the same line). 
Cover 3 cases when NoC ready is zero, random, one during the whole duration of assertion. 

Description:
hpdcache_req_1_valid (ld/st/cmos)  (p_i = 0/1)-> #N mem_req_2_valid  (ld/st)
N = [-5 : 5] @same tag/set
p_i = phys_indexed 
offset between req 1 and req2 have the same or different offsets  
Trigger and clocking events: The clock is core clock. Assertion is trigerred by the request 1 valid. 
Number of Cases: 11x3x2x2x3 = 396 
Note: Covers following cases of on hold request 
1. Cacheable LOAD or PREFETCH, and there is a hit on a pending miss (hit on the MSHR). 
2. Cacheable LOAD or PREFETCH, there is a miss on the cache, and there is a hit (cacheline granularity) on an opened, pending or sent entry of the WBUF 
3. Cacheable STORE, there is a miss on the cache, and there is a hit on a pending miss (hit on the MSHR)
6. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the miss handler FSM cannot send the read miss request

Coverage Method: Assertion Coverage

## Item 271: Shcmoo 5 (hpdcache_req  (@NREQ= X) -> hpdcache_req (@NREQ=Y) )

Test Case: Shcmoo 5 (hpdcache_req  (@NREQ= X) -> hpdcache_req (@NREQ=Y) )

Verification Goal: Purpose: This schmoo is intended to cover the temporal relationship between 2 processor issuing a request at the same time.  If pys_indexed = 0, cache needs 2 cycle to read the address, 1 st cycle it (arbiter) accepts the request and second cycle is used to accept the virtual addresse. 
If another processor makes a request at this second cycle, we have 2 requests coming at the same time. Cover the case where NOC ready always zero, one or random during the duration of assertion
Description:
hpdcache_req_X_valid (ld/st/cmos/amos) (p_i = 0/1) -> #N hpdcache_req_Y_valid (ld/st/cmos/amos)
N = [-5 : 5] 
p_i = phys_indexed 

Trigger and clocking events: The clock is core clock. Assertion is trigerred by the request 1 valid. 
Number of Cases: 11x2x4x4x2 =704

Coverage Method: Assertion Coverage

## Item 272: Schmoo 1 (hpdcache req  -> memory req interface)

Sub Feature: In case 2 (cacheable Load or pre-fetch) which misses in the cache and hits a write-buffer entry which is open closed or sent.

Test Case: Schmoo 1 (hpdcache req  -> memory req interface)

Verification Goal: Purpose: This schmoo is intended to cover the temporal relationship between a processor operation for a cache line and that cache line address appearing on a AXI request  transaction( from a previous request on the same line). 
Cover 3 cases when NoC ready is zero, random, one during the whole duration of assertion. 

Description:
hpdcache_req_1_valid (ld/st/cmos)  (p_i = 0/1)-> #N mem_req_2_valid  (ld/st)
N = [-5 : 5] @same tag/set
p_i = phys_indexed 
offset between req 1 and req2 have the same or different offsets  
Trigger and clocking events: The clock is core clock. Assertion is trigerred by the request 1 valid. 
Number of Cases: 11x3x2x2x3 = 396 
Note: Covers following cases of on hold request 
1. Cacheable LOAD or PREFETCH, and there is a hit on a pending miss (hit on the MSHR). 
2. Cacheable LOAD or PREFETCH, there is a miss on the cache, and there is a hit (cacheline granularity) on an opened, pending or sent entry of the WBUF 
3. Cacheable STORE, there is a miss on the cache, and there is a hit on a pending miss (hit on the MSHR)
6. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the miss handler FSM cannot send the read miss request

Coverage Method: Assertion Coverage

## Item 273: Shcmoo 5 (hpdcache_req  (@NREQ= X) -> hpdcache_req (@NREQ=Y) )

Test Case: Shcmoo 5 (hpdcache_req  (@NREQ= X) -> hpdcache_req (@NREQ=Y) )

Verification Goal: Purpose: This schmoo is intended to cover the temporal relationship between 2 processor issuing a request at the same time.  If pys_indexed = 0, cache needs 2 cycle to read the address, 1 st cycle it (arbiter) accepts the request and second cycle is used to accept the virtual addresse. 
If another processor makes a request at this second cycle, we have 2 requests coming at the same time. Cover the case where NOC ready always zero, one or random during the duration of assertion
Description:
hpdcache_req_X_valid (ld/st/cmos/amos) (p_i = 0/1) -> #N hpdcache_req_Y_valid (ld/st/cmos/amos)
N = [-5 : 5] 
p_i = phys_indexed 

Trigger and clocking events: The clock is core clock. Assertion is trigerred by the request 1 valid. 
Number of Cases: 11x2x4x4x2 =704

Coverage Method: Assertion Coverage

## Item 274: Schmoo 1 (hpdcache req  -> memory req interface)

Sub Feature: In case 3 (cacheable Store) which misses in the cache but matched a pending miss.

Test Case: Schmoo 1 (hpdcache req  -> memory req interface)

Verification Goal: Purpose: This schmoo is intended to cover the temporal relationship between a processor operation for a cache line and that cache line address appearing on a AXI request  transaction( from a previous request on the same line). 
Cover 3 cases when NoC ready is zero, random, one during the whole duration of assertion. 

Description:
hpdcache_req_1_valid (ld/st/cmos)  (p_i = 0/1)-> #N mem_req_2_valid  (ld/st)
N = [-5 : 5] @same tag/set
p_i = phys_indexed 
offset between req 1 and req2 have the same or different offsets  
Trigger and clocking events: The clock is core clock. Assertion is trigerred by the request 1 valid. 
Number of Cases: 11x3x2x2x3 = 396 
Note: Covers following cases of on hold request 
1. Cacheable LOAD or PREFETCH, and there is a hit on a pending miss (hit on the MSHR). 
2. Cacheable LOAD or PREFETCH, there is a miss on the cache, and there is a hit (cacheline granularity) on an opened, pending or sent entry of the WBUF 
3. Cacheable STORE, there is a miss on the cache, and there is a hit on a pending miss (hit on the MSHR)
6. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the miss handler FSM cannot send the read miss request

Coverage Method: Assertion Coverage

## Item 275: Shcmoo 5 (hpdcache_req  (@NREQ= X) -> hpdcache_req (@NREQ=Y) )

Test Case: Shcmoo 5 (hpdcache_req  (@NREQ= X) -> hpdcache_req (@NREQ=Y) )

Verification Goal: Purpose: This schmoo is intended to cover the temporal relationship between 2 processor issuing a request at the same time.  If pys_indexed = 0, cache needs 2 cycle to read the address, 1 st cycle it (arbiter) accepts the request and second cycle is used to accept the virtual addresse. 
If another processor makes a request at this second cycle, we have 2 requests coming at the same time. Cover the case where NOC ready always zero, one or random during the duration of assertion
Description:
hpdcache_req_X_valid (ld/st/cmos/amos) (p_i = 0/1) -> #N hpdcache_req_Y_valid (ld/st/cmos/amos)
N = [-5 : 5] 
p_i = phys_indexed 

Trigger and clocking events: The clock is core clock. Assertion is trigerred by the request 1 valid. 
Number of Cases: 11x2x4x4x2 =704

Coverage Method: Assertion Coverage

## Item 276: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Sub Feature: In case 4 (cacheable Load/Prefetch/Store) hits an entry in the RTAB.

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? HPDCACHE_WBUF_DIR_ENTRIES: 1) +  num_rtab_entry;
        else 
            num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
        end 
        default: 
        begin
          num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES; 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 277: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? HPDCACHE_WBUF_DIR_ENTRIES : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
              default :    num_expected_write =CNT*HPDCACHE_WBUF_DIR_ENTRIES; 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 278: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES): 1) +  num_rtab_entry;
        else 
            num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
        end 
        default: 
        begin
          num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 279: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
              default :    num_expected_write =CNT*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 280: Back Pressure with load at the same address (offset random)

Sub Feature: In case 5 (cacheable Load/Prefetch) and MSHR has no available slots.

Test Case: Back Pressure with load at the same address (offset random)

Verification Goal: Purpose :  The purpose of this test is to verify the number of requests accepted by the MSHR and rtab in the case where acceses are made at the consecutive SETS.  The number of accepted requests depends on rtab_single_flag. 
Target:  MSHR and RTAB mecanism 
Scenerio: In this test, the following sequence  is run with memory reponses disabled for first 1000 cycles. We then check the number of requests accepted by the cache, just before the memory reponses are enabled

    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end

Stimuli: 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :  In this test OP is fixed to HPDCACHE_REQ_LOAD. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets  are random.

The maximum number of requests will be actually dependant of min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS, HPDCACHE_SETS). Here you should replace the 8 by RTAB_ENTRIES

Criteria Pass Fail: Criteria (PASS/FAIL) : 
We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  
Expected number of request the moment back pressure is released after 1000 cycles

get_req_counter() == (m_cfg_rtab_single_entry == 1) ? HPDCACHE_SETS + 1: HPDCACHE_SETS +RTAB_ENTRIES

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 281: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of MSHR, RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be NUM_WBUF_ENTRIES.  If the value does not match the expected a UVM_ERROR is flaged. 
Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 282: Back Pressure with load at the same address (offset random)

Test Case: Back Pressure with load at the same address (offset random)

Verification Goal: Purpose :  The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where acceses are made at the consecutive SETS.  The number of accepted requests depends on rtab_single_flag. 
Target:  MSHR and RTAB mecanism 
Scenerio: In this test, the following sequence  is run with memory reponses disabled for first 1000 cycles. We then check the number of requests accepted by the cache, just before the memory reponses are enabled

    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end

Stimuli: 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :  In this test OP is fixed to HPDCACHE_REQ_LOAD. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets  are random.

The maximum number of requests will be actually dependant of min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS, HPDCACHE_SETS). Here you should replace the 8 by RTAB_ENTRIES

Criteria Pass Fail: Criteria (PASS/FAIL) : 
We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  
Expected number of request the moment back pressure is released after 1000 cycles

get_req_counter() == (m_cfg_rtab_single_entry == 1) ? HPDCACHE_SETS + 1: HPDCACHE_SETS +RTAB_ENTRIES

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 283: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES).  If the value does not match the expected a UVM_ERROR is flaged.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 284: Back Pressure with load at the same address (offset random)

Sub Feature: In case 6 (cacheable Load/Prefetch), miss on the cache and read can not be issued to NoC (congestion).

Test Case: Back Pressure with load at the same address (offset random)

Verification Goal: Purpose :  The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where acceses are made at the consecutive SETS.  The number of accepted requests depends on rtab_single_flag. 
Target:  MSHR and RTAB mecanism 
Scenerio: In this test, the following sequence  is run with memory reponses disabled for first 1000 cycles. We then check the number of requests accepted by the cache, just before the memory reponses are enabled

    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end

Stimuli: 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :  In this test OP is fixed to HPDCACHE_REQ_LOAD. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets  are random.

The maximum number of requests will be actually dependant of min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS, HPDCACHE_SETS). Here you should replace the 8 by RTAB_ENTRIES

Criteria Pass Fail: Criteria (PASS/FAIL) : 
We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  
Expected number of request the moment back pressure is released after 1000 cycles

get_req_counter() == (m_cfg_rtab_single_entry == 1) ? HPDCACHE_SETS + 1: HPDCACHE_SETS +RTAB_ENTRIES

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 285: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES).  If the value does not match the expected a UVM_ERROR is flaged.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 286: Back Pressure with load at the same address (offset random)

Sub Feature: If the RTAB is full, the cache must stop accepting any requests.

Test Case: Back Pressure with load at the same address (offset random)

Verification Goal: Purpose :  The purpose of this test is to verify the number of requests accepted by the MSHR and rtab in the case where acceses are made at the consecutive SETS.  The number of accepted requests depends on rtab_single_flag. 
Target:  MSHR and RTAB mecanism 
Scenerio: In this test, the following sequence  is run with memory reponses disabled for first 1000 cycles. We then check the number of requests accepted by the cache, just before the memory reponses are enabled

    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end

Stimuli: 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :  In this test OP is fixed to HPDCACHE_REQ_LOAD. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets  are random.

The maximum number of requests will be actually dependant of min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS, HPDCACHE_SETS). Here you should replace the 8 by RTAB_ENTRIES

Criteria Pass Fail: Criteria (PASS/FAIL) : 
We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  
Expected number of request the moment back pressure is released after 1000 cycles

get_req_counter() == (m_cfg_rtab_single_entry == 1) ? HPDCACHE_SETS + 1: HPDCACHE_SETS +RTAB_ENTRIES

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 287: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of MSHR, RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be NUM_WBUF_ENTRIES.  If the value does not match the expected a UVM_ERROR is flaged. 
Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 288: Back Pressure with load at the same address (offset random)

Test Case: Back Pressure with load at the same address (offset random)

Verification Goal: Purpose :  The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where acceses are made at the consecutive SETS.  The number of accepted requests depends on rtab_single_flag. 
Target:  MSHR and RTAB mecanism 
Scenerio: In this test, the following sequence  is run with memory reponses disabled for first 1000 cycles. We then check the number of requests accepted by the cache, just before the memory reponses are enabled

    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end

Stimuli: 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :  In this test OP is fixed to HPDCACHE_REQ_LOAD. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets  are random.

The maximum number of requests will be actually dependant of min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS, HPDCACHE_SETS). Here you should replace the 8 by RTAB_ENTRIES

Criteria Pass Fail: Criteria (PASS/FAIL) : 
We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  
Expected number of request the moment back pressure is released after 1000 cycles

get_req_counter() == (m_cfg_rtab_single_entry == 1) ? HPDCACHE_SETS + 1: HPDCACHE_SETS +RTAB_ENTRIES

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 289: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES).  If the value does not match the expected a UVM_ERROR is flaged.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 290: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Sub Feature: When requests are taken out of the RTAB, they must be taken out in an order that respects the RVWMO. Specifically, if two requests in the RTAB overlap, then they must be taken out in the order they entered.

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? HPDCACHE_WBUF_DIR_ENTRIES: 1) +  num_rtab_entry;
        else 
            num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
        end 
        default: 
        begin
          num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES; 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 291: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? HPDCACHE_WBUF_DIR_ENTRIES : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
              default :    num_expected_write =CNT*HPDCACHE_WBUF_DIR_ENTRIES; 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 292: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES): 1) +  num_rtab_entry;
        else 
            num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
        end 
        default: 
        begin
          num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 293: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
              default :    num_expected_write =CNT*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 294 - Requirement 26.0: test_hpdcache_multiple_random_requests

Requirement ID: 26.0

Feature: The cache must be dead-lock free.

Sub Feature: No sequence of events entering the replay-buffer must trigger a dead-lock or live-lock condition.

Feature Description: The cache must be dead-lock free.

Test Case: test_hpdcache_multiple_random_requests

Verification Goal: Purpose : Test to perform random accesses. The purpose is to have maximum coverage(functional and code).  
Target     : It targets every feature of the cache. 
Scenario: Random
Stimuli : This test  respects the constraints related to hpdcache protocol
Requirement : 
Constraint      :  Every field of the transaction is randomized. Reserved op code is not used. The address are biased to more frequently select the extreme tag values(0x0, 0xFFFFF ….). The distribution is biased to have more request with small delays between requests.
Cacheable requests are previleged over uncacheable requests (95% cacheable / 5% uncacheable). Uncacheable requests empty the pipeline, hence limiting concurrency. Concurrency is the best way to detect possible deadlocks/livelocks.
"Normal" accesses (load, stores) are previliged over CMOs and AMOs (90% normal / 5% CMOs and AMOs)

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 295: test_hpdcache_multiple_random_requests_uncached

Test Case: test_hpdcache_multiple_random_requests_uncached

Verification Goal: This test is derived from test 1.1.1. 
Constraint       :  The field uncacheable is constrained  to have only uncacheable accesses. And is added for coverage.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 296: test_hpdcache_multiple_load_store_requests

Test Case: test_hpdcache_multiple_load_store_requests

Verification Goal: This test is derived from test 1.1.1. 
Stimuli:   In this test 80% of the traffic is load and store.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 297: test_hpdcache_multiple_random_requests_in_region

Test Case: test_hpdcache_multiple_random_requests_in_region

Verification Goal: Purpose : Test to perform random accesses within randomized address  regions. The purpose is to provoke hits and conflicts keeping stimulus relatively random. 
Target     : It targets every feature of the cache. 
Scenario: Random
Stimuli : This test  respects the constraints related to hpdcache protocol
Requirement :  The regions are generated using memory partition class from cv_dv_utils. The regions are random size but some are small which provokes hits and conflicts.
The sequence choses one region randomly for a SEED.
Constraint       :  Every field of the transaction is randomized. Reserved op code is not used. The addresses are generated within the region. The distribution is biased to have more request with small delays between requests.
Cacheable requests are previleged over uncacheable requests (95% cacheable / 5% uncacheable). Uncacheable requests empty the pipeline, hence limiting concurrency. Concurrency is the best way to detect possible deadlocks/livelocks.
"Normal" accesses (load, stores) are previliged over CMOs and AMOs (90% normal / 5% CMOs and AMOs)

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 298: test_hpdcache_multiple_load_store_requests_in_region

Test Case: test_hpdcache_multiple_load_store_requests_in_region

Verification Goal: Same as 2.1.4. 
Stimuli: This test performs  80% load store accesses within randomized address regions and the other accesses are fully random but within the regions.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 299: test_hpdcache_multiple_directed_addr

Test Case: test_hpdcache_multiple_directed_addr

Verification Goal: Purpose :  Test to provoke the eviction at regular interval in between many hits.
Target     :  This test stresses the eviction mechanism. 
Scenario:  In this test, one set is selected randomly at the beginning of the test.  Then NWAYS + 1 random tags are selected within the selected set. Random loads  and stores are performed on these preselected {tag, set}. Normally we should observe many hits with misses (because of the evictions) as the number of tags exceed the NWAYS by 1.  
Stimuli :  The test runs hpdcache_multiple_directed_addr sequence which is driven from hpdcache_single_directed_addr. 
Requirement :  This test  respects the constraints related to hpdcache protocol
Constraint       :  Every field of the transaction is randomized except set, address and uncacheable (always 0 -> cacheable).

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 300: test_hpdcache_multiple_directed_addr_bPLRU_prediction

Test Case: test_hpdcache_multiple_directed_addr_bPLRU_prediction

Verification Goal: Purpose:  The purpose of this the test is to check the PLRU eviction algorithm using a black box approach.
Target:  PLRU algorithm 
Scenario:  To avoid temporal conflicts between request and refill response which cannot be predicted, in this test each request is sent every N( = 15) cycles and the memory response model is configured to respond with zero delay. We require that the response from the previous request is received before the next request is issued. Otherwise it is impossible to predict the PLRU behavior using a black box approach.
Stimuli :  The sequence is derived from 1.2.1
Requirement :  This test  respects the constraints related to hpdcache protocol. This test enables a UVM based model in the SB which  does the black box prediction of the PLRU under a limited set of stimulus which is respected by this test.
Constraint       :  Every field of the transaction is randomized except Set, address, uncacheable (always 0 -> cacheable) and inter request delay. Memory response model is configured to respond with zero delay. 
Note:  We note that the environment also has a SV based PLRU model which is bind directly to the memory control module to do the exact prediction of the PLRU using the signals from memory control module (white box assertion based checking).   In the UVM based model, the PLRU prediction takes into accoun the type of transactions.

Criteria Pass Fail: The TB has UVM based PLRU model which flags a UVM_ERROR in case PLRU algorithm is not respected. A tag directory is maintained which helps predict hit and miss in this particular scenario.

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 301: test_hpdcache_multiple_random_requests_in_region_bPLRU_prediction

Test Case: test_hpdcache_multiple_random_requests_in_region_bPLRU_prediction

Verification Goal: This is the variant of test 1.2.2 where this test is derived from the test 1.1.4.
Scenario: The idea is to perform black box PLRU prediction when the memory accesses are distributed across a memory region. In 1.2.2 the accesses where focused on a single set to provoke frequent evictions.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 302: test_hpdcache_multiple_amo_lr_sc_requests

Test Case: test_hpdcache_multiple_amo_lr_sc_requests

Verification Goal: Purpose: The aim of this test is to test Load Reserved (LR)/Store Conditional (SC) sequences. The goal is to test all possible conditions where LR/SC fails or passes. A coverage item is added to cover all possible combination. A sequence of LR/SC mixed with LOAD/STORE/AMOS/CMOS are run. Most(90%) of the accesses are done at the same address.
Target: Load Reserved and Store Conditional Mechanism 
Scenario:  The hpdcache_single_lr_sc_request runs a sequence with following constraints: 
FOR 1 to (random 10 or 15 )
          m_req_addr dist { same_addr := (50 a 100), random_addr := (100 - same_addr)};
          m_req_op   dist { hpdcache_REQ_AMO_LR := 40, HPDCACHE_REQ_AMO_SC := 40, 
                            hpdcache_REQ_LOAD     := 11,
                            hpdcache_REQ_AMO_SWAP := 1,
                            hpdcache_REQ_STORE  := 1, 
                            hpdcache_REQ_AMO_ADD  := 1,
                            hpdcache_REQ_AMO_AND  := 1,
                            hpdcache_REQ_AMO_OR   := 1,
                            hpdcache_REQ_AMO_XOR  := 1,
                            hpdcache_REQ_AMO_MAX  := 1,
                            hpdcache_REQ_AMO_MAXU := 1,
                            hpdcache_REQ_AMO_MIN  := 1,
                            hpdcache_REQ_AMO_MINU := 1
                            hpdcache_REQ_CMO := 1 
END
Stimuli: The sequence used is hpdcache_multiple_amo_lr_sc_requests which is driven from hpdcache_single_lr_sc_request.
Requirement :  This test  respects the constraints related to hpdcache protocol. 
Constraint:  Every field of the transaction is randomized except address.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 303: test_reg_bit_bash

Test Case: test_reg_bit_bash

Verification Goal: Built-in UVM sequences to check CSRs via reg model

Criteria Pass Fail: UVM regmodel flags UVM_ERROR in case of a mismatch

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 304: test reg access sequence

Test Case: test reg access sequence

Verification Goal: Built-in UVM sequences to check CSRs via reg model

Criteria Pass Fail: UVM regmodel flags UVM_ERROR in case of a mismatch

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 305: test reg hw reset register test

Test Case: test reg hw reset register test

Verification Goal: Built-in UVM sequences to check CSRs via reg model

Criteria Pass Fail: UVM regmodel flags UVM_ERROR in case of a mismatch

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 306: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? HPDCACHE_WBUF_DIR_ENTRIES: 1) +  num_rtab_entry;
        else 
            num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
        end 
        default: 
        begin
          num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES; 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 307: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? HPDCACHE_WBUF_DIR_ENTRIES : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
              default :    num_expected_write =CNT*HPDCACHE_WBUF_DIR_ENTRIES; 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 308: Back Pressure with load at the same address (offset random)

Test Case: Back Pressure with load at the same address (offset random)

Verification Goal: Purpose :  The purpose of this test is to verify the number of requests accepted by the MSHR and rtab in the case where acceses are made at the consecutive SETS.  The number of accepted requests depends on rtab_single_flag. 
Target:  MSHR and RTAB mecanism 
Scenerio: In this test, the following sequence  is run with memory reponses disabled for first 1000 cycles. We then check the number of requests accepted by the cache, just before the memory reponses are enabled

    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end

Stimuli: 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :  In this test OP is fixed to HPDCACHE_REQ_LOAD. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets  are random.

The maximum number of requests will be actually dependant of min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS, HPDCACHE_SETS). Here you should replace the 8 by RTAB_ENTRIES

Criteria Pass Fail: Criteria (PASS/FAIL) : 
We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  
Expected number of request the moment back pressure is released after 1000 cycles

get_req_counter() == (m_cfg_rtab_single_entry == 1) ? HPDCACHE_SETS + 1: HPDCACHE_SETS +RTAB_ENTRIES

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 309: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of MSHR, RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be NUM_WBUF_ENTRIES.  If the value does not match the expected a UVM_ERROR is flaged. 
Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 310: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES): 1) +  num_rtab_entry;
        else 
            num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
        end 
        default: 
        begin
          num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 311: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
              default :    num_expected_write =CNT*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 312: Back Pressure with load at the same address (offset random)

Test Case: Back Pressure with load at the same address (offset random)

Verification Goal: Purpose :  The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where acceses are made at the consecutive SETS.  The number of accepted requests depends on rtab_single_flag. 
Target:  MSHR and RTAB mecanism 
Scenerio: In this test, the following sequence  is run with memory reponses disabled for first 1000 cycles. We then check the number of requests accepted by the cache, just before the memory reponses are enabled

    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end

Stimuli: 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :  In this test OP is fixed to HPDCACHE_REQ_LOAD. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets  are random.

The maximum number of requests will be actually dependant of min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS, HPDCACHE_SETS). Here you should replace the 8 by RTAB_ENTRIES

Criteria Pass Fail: Criteria (PASS/FAIL) : 
We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  
Expected number of request the moment back pressure is released after 1000 cycles

get_req_counter() == (m_cfg_rtab_single_entry == 1) ? HPDCACHE_SETS + 1: HPDCACHE_SETS +RTAB_ENTRIES

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 313: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES).  If the value does not match the expected a UVM_ERROR is flaged.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 314: Write only with limited back pressure

Test Case: Write only with limited back pressure

Verification Goal: Purpose :  The aim of this performance test is to show the benefits of having a write buffer on the performance. 
Check the bandwidth of cached write at the core interface of HPDcache when writes arrive with no inter request delay. 
If memory response reply within a delay < NUM_WBUF cycles, the cache should be able to accept every write. 
Target     : Write Performance 
Scenario:  Write only test. 
Stimuli   :  Test is derived from random access tests.  
Requirement : 
Following condition should be respected to get full performance 
1. Store accesses are cacheable 
2. There is not inter request delay @core interface 
3. Memory response model reply within minimum (< NUM_WBUF cycles) delay possible 

Constraints: 
OP = HPDCACHE_REQ_STORE, uncacheable=0. 
The wbuf and rtab should be configured to get the maximum output. Following values are used:
                 m_cfg_wbuf_threshold = > 2 < HPDCACHE_WBUF_DIR_ENTRIES
                 m_cfg_wbuf_reset_timecnt_on_write = 1;
                 m_cfg_wbuf_sequential_waw = 0;
                 m_cfg_wbuf_inhibit_write_coalescing = 0;
                 m_cfg_rtab_single_entry = 0

All writes are done at unique SET where no request is on the fly.

Criteria Pass Fail: 
Criteria (PASS/FAIL): 
A check is performed at the end of test in report phase.
The cache is expected to accept  NB_TXN stores in NB_TXN cycles. 

In the case where write_buf_dir_entry, write_buf_data_entry < the memory write latency,  almost 3 times degrdation of performance is observed.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 315: Read only with no memory back pressure

Test Case: Read only with no memory back pressure

Verification Goal: Purpose:  The aim of this performance test is to show the benefits of having a MSHR on the performance. 
Check the bandwidth of cached reads at the core interface of HPDcache when a read arrives with no inter request delay. 
If memory response reply within a cycle (or may be cycles < MSHR_SET), the cache should be able to accept a read miss every three cycle. 
Target    : Load Performance maximising the use of MSHR 
Scenario : Load only test
Stimulie : Test uses random access sequence. 
Requirement: 
Following condition are respected to get full performance 
1. Load accesses are cacheable 
2. There is no conflit within the MSHR (An access should not be made on a SET which already has an ongoing miss) 
3. There is no inter request delay @core interface 
4. Memory response model reply within minimum delay possible 
Constraint: 
OP = HPDCACHE_REQ_LOAD, uncacheable=0. 
The wbuf and rtab should be configurered to get the maximum outuput. Following values are used:
                 m_cfg_rtab_single_entry = 0

Criteria Pass Fail: Criteria (PASS/FAIL): 
A check is performed at the end of test in report phase. 
The cache is expected to accept  NB_TXN loads in 3*NB_TXN cycles.
Note: Here, when you say SET, it should be MSHR_SET (which may be different than the number of sets in the cache HPDCACHE_SETS). Another thing, is that you still need to repeat the MSHR_SET in order to exploit all the MSHR as the MSHR has MSHR_WAYS per MSHR_SET. Thus, I do not know how you generate the addresses, maybe it is already as I propose hereafter:

req.address = (random() / CL_BYTES) * CL_BYTES            // align the base address to a cacheline
while (true) {
    req.address = req.address + CL_BYTES
}

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 316: Read only with large memory back pressure

Test Case: Read only with large memory back pressure

Verification Goal: Purpose: The aim of this performance test is to show the benefits of having a MSHR on the performance. 
Check the bandwidth of cached reads at the core interface of HPDcache when a read arrives with no inter request delay. 
If memory response reply with a delay of 2*MSHR_SETS*MSHR_WAYS cycle(more realistic delay), the cache should be able to accept a read miss every third cycle. 
Tarrget:    Load Performance maximising the use of MSHR 
Scenario: Load only test
Stimulie: Test uses random access sequence. 
Requirements: 
Following condition are respected to get full performance 
1. Load accesses are cacheable 
2. There is no conflit within the MSHR (An access should not be made on a SET which already has an ongoing miss) 
3. There is no inter request delay @core interface 
4. Memory response model reply within minimum delay possible 
Constraints: 
The wbuf and rtab should be configurered to get the maximum outuput. Following values are used:
                 m_cfg_rtab_single_entry = 0

Criteria Pass Fail: Criteria (PASS/FAIL): 
A check is performed at the end of test in report phase. 
The cache is expected to accept  NB_TXN loads in 2*3*NB_TXN cycles.
I would generate the address as above (cf 2.5.2)

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 317: Write(30%) Read(70% ) with no memory back pressure

Test Case: Write(30%) Read(70% ) with no memory back pressure

Verification Goal: Purpose : Check the bandwidth of cached reads (70%) and cached writes(30%) at the core interface of HPDcache when reads and writes arrive with no inter request delay. 
Target     : Read and Write Performance 
Scenario:  Test is constraint to have 70% cacheable read and 30% cacheable  write. 
Stimuli   :  Test is derived from random access tests.  
Requirements: 
Following condition are respected to get full performance 
1. Load/Store accesses are cacheable 
2. There is no conflit within the MSHR (An access should not be made on a SET which already has an ongoing miss) 
3. There is no inter request delay @core interface 
4. Memory response model reply within minimum delay possible 
Constraints: 
OP = HPDCACHE_REQ_LOAD(70%), HPDCACHE_REQ_STORE(30%). Uncacheable = 0. 
5. The wbuf and rtab should be configurered to get the maximum outuput. Following values are used:
                 m_cfg_wbuf_threshold = > 2
                 m_cfg_wbuf_reset_timecnt_on_write = 1;
                 m_cfg_wbuf_sequential_waw = 0;
                 m_cfg_wbuf_inhibit_write_coalescing = 0;
                 m_cfg_rtab_single_entry = 0

Criteria Pass Fail: A check is performed at the end of test in report phase. 
The cache is expected to accept  NB_TXN loads/stores in 3*NB_TXN cycles.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 318 - Requirement 27.0: test_hpdcache_multiple_random_requests

Requirement ID: 27.0

Feature: The cache must be live-lock free.

Feature Description: The cache must be live-lock free.

Test Case: test_hpdcache_multiple_random_requests

Verification Goal: Purpose : Test to perform random accesses. The purpose is to have maximum coverage(functional and code).  
Target     : It targets every feature of the cache. 
Scenario: Random
Stimuli : This test  respects the constraints related to hpdcache protocol
Requirement : 
Constraint      :  Every field of the transaction is randomized. Reserved op code is not used. The address are biased to more frequently select the extreme tag values(0x0, 0xFFFFF ….). The distribution is biased to have more request with small delays between requests.
Cacheable requests are previleged over uncacheable requests (95% cacheable / 5% uncacheable). Uncacheable requests empty the pipeline, hence limiting concurrency. Concurrency is the best way to detect possible deadlocks/livelocks.
"Normal" accesses (load, stores) are previliged over CMOs and AMOs (90% normal / 5% CMOs and AMOs)

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 319: test_hpdcache_multiple_random_requests_uncached

Test Case: test_hpdcache_multiple_random_requests_uncached

Verification Goal: This test is derived from test 1.1.1. 
Constraint       :  The field uncacheable is constrained  to have only uncacheable accesses. And is added for coverage.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 320: test_hpdcache_multiple_load_store_requests

Test Case: test_hpdcache_multiple_load_store_requests

Verification Goal: This test is derived from test 1.1.1. 
Stimuli:   In this test 80% of the traffic is load and store.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 321: test_hpdcache_multiple_random_requests_in_region

Test Case: test_hpdcache_multiple_random_requests_in_region

Verification Goal: Purpose : Test to perform random accesses within randomized address  regions. The purpose is to provoke hits and conflicts keeping stimulus relatively random. 
Target     : It targets every feature of the cache. 
Scenario: Random
Stimuli : This test  respects the constraints related to hpdcache protocol
Requirement :  The regions are generated using memory partition class from cv_dv_utils. The regions are random size but some are small which provokes hits and conflicts.
The sequence choses one region randomly for a SEED.
Constraint       :  Every field of the transaction is randomized. Reserved op code is not used. The addresses are generated within the region. The distribution is biased to have more request with small delays between requests.
Cacheable requests are previleged over uncacheable requests (95% cacheable / 5% uncacheable). Uncacheable requests empty the pipeline, hence limiting concurrency. Concurrency is the best way to detect possible deadlocks/livelocks.
"Normal" accesses (load, stores) are previliged over CMOs and AMOs (90% normal / 5% CMOs and AMOs)

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 322: test_hpdcache_multiple_load_store_requests_in_region

Test Case: test_hpdcache_multiple_load_store_requests_in_region

Verification Goal: Same as 2.1.4. 
Stimuli: This test performs  80% load store accesses within randomized address regions and the other accesses are fully random but within the regions.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 323: test_hpdcache_multiple_directed_addr

Test Case: test_hpdcache_multiple_directed_addr

Verification Goal: Purpose :  Test to provoke the eviction at regular interval in between many hits.
Target     :  This test stresses the eviction mechanism. 
Scenario:  In this test, one set is selected randomly at the beginning of the test.  Then NWAYS + 1 random tags are selected within the selected set. Random loads  and stores are performed on these preselected {tag, set}. Normally we should observe many hits with misses (because of the evictions) as the number of tags exceed the NWAYS by 1.  
Stimuli :  The test runs hpdcache_multiple_directed_addr sequence which is driven from hpdcache_single_directed_addr. 
Requirement :  This test  respects the constraints related to hpdcache protocol
Constraint       :  Every field of the transaction is randomized except set, address and uncacheable (always 0 -> cacheable).

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 324: test_hpdcache_multiple_directed_addr_bPLRU_prediction

Test Case: test_hpdcache_multiple_directed_addr_bPLRU_prediction

Verification Goal: Purpose:  The purpose of this the test is to check the PLRU eviction algorithm using a black box approach.
Target:  PLRU algorithm 
Scenario:  To avoid temporal conflicts between request and refill response which cannot be predicted, in this test each request is sent every N( = 15) cycles and the memory response model is configured to respond with zero delay. We require that the response from the previous request is received before the next request is issued. Otherwise it is impossible to predict the PLRU behavior using a black box approach.
Stimuli :  The sequence is derived from 1.2.1
Requirement :  This test  respects the constraints related to hpdcache protocol. This test enables a UVM based model in the SB which  does the black box prediction of the PLRU under a limited set of stimulus which is respected by this test.
Constraint       :  Every field of the transaction is randomized except Set, address, uncacheable (always 0 -> cacheable) and inter request delay. Memory response model is configured to respond with zero delay. 
Note:  We note that the environment also has a SV based PLRU model which is bind directly to the memory control module to do the exact prediction of the PLRU using the signals from memory control module (white box assertion based checking).   In the UVM based model, the PLRU prediction takes into accoun the type of transactions.

Criteria Pass Fail: The TB has UVM based PLRU model which flags a UVM_ERROR in case PLRU algorithm is not respected. A tag directory is maintained which helps predict hit and miss in this particular scenario.

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 325: test_hpdcache_multiple_random_requests_in_region_bPLRU_prediction

Test Case: test_hpdcache_multiple_random_requests_in_region_bPLRU_prediction

Verification Goal: This is the variant of test 1.2.2 where this test is derived from the test 1.1.4.
Scenario: The idea is to perform black box PLRU prediction when the memory accesses are distributed across a memory region. In 1.2.2 the accesses where focused on a single set to provoke frequent evictions.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 326: test_hpdcache_multiple_amo_lr_sc_requests

Test Case: test_hpdcache_multiple_amo_lr_sc_requests

Verification Goal: Purpose: The aim of this test is to test Load Reserved (LR)/Store Conditional (SC) sequences. The goal is to test all possible conditions where LR/SC fails or passes. A coverage item is added to cover all possible combination. A sequence of LR/SC mixed with LOAD/STORE/AMOS/CMOS are run. Most(90%) of the accesses are done at the same address.
Target: Load Reserved and Store Conditional Mechanism 
Scenario:  The hpdcache_single_lr_sc_request runs a sequence with following constraints: 
FOR 1 to (random 10 or 15 )
          m_req_addr dist { same_addr := (50 a 100), random_addr := (100 - same_addr)};
          m_req_op   dist { hpdcache_REQ_AMO_LR := 40, HPDCACHE_REQ_AMO_SC := 40, 
                            hpdcache_REQ_LOAD     := 11,
                            hpdcache_REQ_AMO_SWAP := 1,
                            hpdcache_REQ_STORE  := 1, 
                            hpdcache_REQ_AMO_ADD  := 1,
                            hpdcache_REQ_AMO_AND  := 1,
                            hpdcache_REQ_AMO_OR   := 1,
                            hpdcache_REQ_AMO_XOR  := 1,
                            hpdcache_REQ_AMO_MAX  := 1,
                            hpdcache_REQ_AMO_MAXU := 1,
                            hpdcache_REQ_AMO_MIN  := 1,
                            hpdcache_REQ_AMO_MINU := 1
                            hpdcache_REQ_CMO := 1 
END
Stimuli: The sequence used is hpdcache_multiple_amo_lr_sc_requests which is driven from hpdcache_single_lr_sc_request.
Requirement :  This test  respects the constraints related to hpdcache protocol. 
Constraint:  Every field of the transaction is randomized except address.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 327: test_reg_bit_bash

Test Case: test_reg_bit_bash

Verification Goal: Built-in UVM sequences to check CSRs via reg model

Criteria Pass Fail: UVM regmodel flags UVM_ERROR in case of a mismatch

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 328: test reg access sequence

Test Case: test reg access sequence

Verification Goal: Built-in UVM sequences to check CSRs via reg model

Criteria Pass Fail: UVM regmodel flags UVM_ERROR in case of a mismatch

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 329: test reg hw reset register test

Test Case: test reg hw reset register test

Verification Goal: Built-in UVM sequences to check CSRs via reg model

Criteria Pass Fail: UVM regmodel flags UVM_ERROR in case of a mismatch

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 330: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? HPDCACHE_WBUF_DIR_ENTRIES: 1) +  num_rtab_entry;
        else 
            num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
        end 
        default: 
        begin
          num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES; 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 331: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? HPDCACHE_WBUF_DIR_ENTRIES : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*HPDCACHE_WBUF_DIR_ENTRIES  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = HPDCACHE_WBUF_DIR_ENTRIES;
              default :    num_expected_write =CNT*HPDCACHE_WBUF_DIR_ENTRIES; 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 332: Back Pressure with load at the same address (offset random)

Test Case: Back Pressure with load at the same address (offset random)

Verification Goal: Purpose :  The purpose of this test is to verify the number of requests accepted by the MSHR and rtab in the case where acceses are made at the consecutive SETS.  The number of accepted requests depends on rtab_single_flag. 
Target:  MSHR and RTAB mecanism 
Scenerio: In this test, the following sequence  is run with memory reponses disabled for first 1000 cycles. We then check the number of requests accepted by the cache, just before the memory reponses are enabled

    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end

Stimuli: 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :  In this test OP is fixed to HPDCACHE_REQ_LOAD. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets  are random.

The maximum number of requests will be actually dependant of min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS, HPDCACHE_SETS). Here you should replace the 8 by RTAB_ENTRIES

Criteria Pass Fail: Criteria (PASS/FAIL) : 
We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  
Expected number of request the moment back pressure is released after 1000 cycles

get_req_counter() == (m_cfg_rtab_single_entry == 1) ? HPDCACHE_SETS + 1: HPDCACHE_SETS +RTAB_ENTRIES

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 333: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of MSHR, RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be NUM_WBUF_ENTRIES.  If the value does not match the expected a UVM_ERROR is flaged. 
Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 334: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter enable

Verification Goal: Purpose: The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where accesses are made at the same SET. The number of accepted requests depends on wbuf_threshold, rtab_single_flag, wbuf_waw_flag, wbuf_inhibit_write_coalescing, cfg_wbuf_reset_timecnt_on_write_i
Target:  Write Buffer and RTAB. 
Scenario:  This sequence issues only HPDCACHE_REQ_DATA_WIDTH bits wide cacheable STORE at the same cache line with different offsets. Inter request delay is zero and only one requester is enabled at a time. 
 cnt = $urandom_range(env.m_hpdcache_conf.m_cfg_wbuf_threshold+1, env.m_hpdcache_conf.m_cfg_wbuf_threshold+10);

For i=1->HPDCACHE_WBUF_DIR_ENTRIES+HPDCACHE_RTAB_ENTRIES+10
    tag = random;
    set = unique_set[i];
    For j=1->CNT 
     offset=random
     addr = {tag, set, offset}
     cacheable=1
     op = STORE 
   End 
End
Stimuli: The sequence is hpdcache_same_tag_set_access_request_cached
Requirements:  In this test, the sequence  is run with memory responses disabled at first (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. The memory response model accepts each request but does not respond. 
Constraint:  In this test m_cfg_wbuf_reset_timecnt_on_write is enabled to 1. OP is fixed to HPDCACHE_REQ_STORE. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets are random.
Note: Every time a write is observed the wbuf time counter is reset to 0. There is no delay between the requests made to hpdcache. Hence write buffer counter will be reseted all the time.

Criteria Pass Fail: Criteria (PASS/FAIL): We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number of write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*CNT+HPDCACHE_RTAB_ENTRIES) cycles. 

num_rtab_entry = (m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES );
if wbuf_inhibit_write_coalescing = 1 // write merge not allowed
        if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin
           num_expected_write =( (cnt==1) ? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES): 1) +  num_rtab_entry;
        else 
            num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry 
       end
else 

  if(env.m_hpdcache_conf.m_cfg_wbuf_sequential_waw == 1 ) begin // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
     num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
  end else begin
      case(env.m_hpdcache_conf.m_cfg_wbuf_threshold) 
        0: 
        begin
          num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
        end 
        default: 
        begin
          num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
        end
      endcase
      num_expected_write =  num_expected_write + ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES)); 
    end

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 335: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Test Case: Back Pressure with Write HPDCACHE_REQ_DATA_WIDTH bits same address (offset random) with threshold reset time counter disable

Verification Goal: Test is same as 2.3.1

Constraint                     :  In this test m_cfg_wbuf_reset_timecnt_on_write is disabled.

Criteria Pass Fail: Criteria (PASS/FAIL)  : We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  Following formula is used to check the expected number write requests accepted by HPDCACHE in the first  (HPDCACHE_WBUF_DIR_ENTRIES*(m_cfg_wbuf_threshold*2+1)+HPDCACHE_RTAB_ENTRIES) cycles. 
   CNT  = (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 0) ? $urandom_range(1, 2) : 
                 (env.m_hpdcache_conf.m_cfg_wbuf_threshold == 1) ? $urandom_range(1, 2) : 
                  $urandom_range(1, env.m_hpdcache_conf.m_cfg_wbuf_threshold);

num_rtab_entry             = ((m_cfg_rtab_single_entry == 1) ? 1 : (HPDCACHE_RTAB_ENTRIES));


if(m_cfg_wbuf_sequential_waw == 1 ) begin  // Cannot have 2 writes on the fly at the same address after wbuf SENT (merging is possible)
   if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin // mergin is not allowed in the wbuf
        num_expected_write = ((cnt ==1)? MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) : 1) +  num_rtab_entry;
   end else begin
        num_expected_write =  cnt*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES)  + num_rtab_entry; 
    end
else // no waw=0
     if(env.m_hpdcache_conf.m_cfg_wbuf_inhibit_write_coalescing == 1) begin
           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES) +  num_rtab_entry;
       else
        CASE m_cfg_wbuf_threshold
              [0:1]:           num_expected_write = MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES);
              default :    num_expected_write =CNT*MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES); 
            endcase
               num_expected_write =  num_expected_write +num_rtab_entry
         end
Note: HPDcache has 3 pipeline stage, so a fuzzyness of 3 expected in the expected write

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 336: Back Pressure with load at the same address (offset random)

Test Case: Back Pressure with load at the same address (offset random)

Verification Goal: Purpose :  The purpose of this test is to verify the number of requests accepted by the wbuf and rtab in the case where acceses are made at the consecutive SETS.  The number of accepted requests depends on rtab_single_flag. 
Target:  MSHR and RTAB mecanism 
Scenerio: In this test, the following sequence  is run with memory reponses disabled for first 1000 cycles. We then check the number of requests accepted by the cache, just before the memory reponses are enabled

    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end

Stimuli: 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :  In this test OP is fixed to HPDCACHE_REQ_LOAD. Uncacheable = 0 -> always cacheable. SETs are choses from a UNIQUE set list. TAGs and offsets  are random.

The maximum number of requests will be actually dependant of min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS, HPDCACHE_SETS). Here you should replace the 8 by RTAB_ENTRIES

Criteria Pass Fail: Criteria (PASS/FAIL) : 
We check the number of requests accepted by the cache, just before the memory reponses are enabled. If the value does not match the expected a UVM_ERROR is flaged.  
Expected number of request the moment back pressure is released after 1000 cycles

get_req_counter() == (m_cfg_rtab_single_entry == 1) ? HPDCACHE_SETS + 1: HPDCACHE_SETS +RTAB_ENTRIES

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 337: Back Pressure with LOAD
Back Pressure with STORE

Test Case: Back Pressure with LOAD
Back Pressure with STORE

Verification Goal: Purpose :  The purpose of this test is to check the functionning of RTAB and Write Buffer in the case accesses are made at the different SETs(unique). 
Target:  MSHR, RTAB and Write Buffer mechanism 
Scenario :  In this tests following sequence is run with memory responses disabled for first 1000 cycles. 
In this test only one requester is enabled.
    for(int i = 0; i < num_txn; i ++) begin

      // --------------------------------
      // Randomize transaction item
      // --------------------------------
      if ( !item.randomize() with { m_req_sid == sid ; } )
        `uvm_fatal("body","Randomization failed");

      set    = hpdcache_get_req_addr_set(item.m_req_addr);
      offset = hpdcache_get_req_addr_offset(item.m_req_addr);
      tag    = hpdcache_get_req_addr_tag(item.m_req_addr);

      if(i < HPDCACHE_SETS) begin
        set = unique_set[i]; 
        item.m_req_addr = {tag, set, offset};
      end
    end
Stimuli               : The sequence is hpdcache_consecutive_set_access_request_cached. 
Requirements:  In this test, the sequence  is run with memory reponses disabled for first 1000 cycles. The memory response model accepts each request but does not respond. 
Constraint        :   In this test OP is fixed to HPDCACHE_REQ_LOAD/STORE. Uncacheable = 0 -> alwasy cacheable. SETs are UNIQUE in the first 1000 cycles. TAGs and offsets are random.

Note: Covers the following case of on hold request 
5. Cacheable LOAD or PREFETCH, there is a miss on the cache, and the MSHR has no available slots
As above, the number of loads shall be min(HPDCACHE_MSHR_SETS*HPDCACHE_MSHR_WAYS,HPDCACHE_SETS)

Criteria Pass Fail: Criteria (PASS/FAIL)  : In this test in the case of LOAD we get minimum (SETS + RTAB ENTRIES) number of miss on the fly. In the case of the store, requests on fly are expected to be MIN (HPDCACHE_WBUF_DIR_ENTRIES, HPDCACHE_WBUF_DATA_ENTRIES).  If the value does not match the expected a UVM_ERROR is flaged.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 338: Write only with limited back pressure

Test Case: Write only with limited back pressure

Verification Goal: Purpose :  The aim of this performance test is to show the benefits of having a write buffer on the performance. 
Check the bandwidth of cached write at the core interface of HPDcache when writes arrive with no inter request delay. 
If memory response reply within a delay < NUM_WBUF cycles, the cache should be able to accept every write. 
Target     : Write Performance 
Scenario:  Write only test. 
Stimuli   :  Test is derived from random access tests.  
Requirement : 
Following condition should be respected to get full performance 
1. Store accesses are cacheable 
2. There is not inter request delay @core interface 
3. Memory response model reply within minimum (< NUM_WBUF cycles) delay possible 

Constraints: 
OP = HPDCACHE_REQ_STORE, uncacheable=0. 
The wbuf and rtab should be configured to get the maximum output. Following values are used:
                 m_cfg_wbuf_threshold = > 2 < HPDCACHE_WBUF_DIR_ENTRIES
                 m_cfg_wbuf_reset_timecnt_on_write = 1;
                 m_cfg_wbuf_sequential_waw = 0;
                 m_cfg_wbuf_inhibit_write_coalescing = 0;
                 m_cfg_rtab_single_entry = 0

All writes are done at unique SET where no request is on the fly.

Criteria Pass Fail: 
Criteria (PASS/FAIL): 
A check is performed at the end of test in report phase.
The cache is expected to accept  NB_TXN stores in NB_TXN cycles. 

In the case where write_buf_dir_entry, write_buf_data_entry < the memory write latency,  almost 3 times degrdation of performance is observed.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 339: Read only with no memory back pressure

Test Case: Read only with no memory back pressure

Verification Goal: Purpose:  The aim of this performance test is to show the benefits of having a MSHR on the performance. 
Check the bandwidth of cached reads at the core interface of HPDcache when a read arrives with no inter request delay. 
If memory response reply within a cycle (or may be cycles < MSHR_SET), the cache should be able to accept a read miss every three cycle. 
Target    : Load Performance maximising the use of MSHR 
Scenario : Load only test
Stimulie : Test uses random access sequence. 
Requirement: 
Following condition are respected to get full performance 
1. Load accesses are cacheable 
2. There is no conflit within the MSHR (An access should not be made on a SET which already has an ongoing miss) 
3. There is no inter request delay @core interface 
4. Memory response model reply within minimum delay possible 
Constraint: 
OP = HPDCACHE_REQ_LOAD, uncacheable=0. 
The wbuf and rtab should be configurered to get the maximum outuput. Following values are used:
                 m_cfg_rtab_single_entry = 0

Criteria Pass Fail: Criteria (PASS/FAIL): 
A check is performed at the end of test in report phase. 
The cache is expected to accept  NB_TXN loads in 3*NB_TXN cycles.
Note: Here, when you say SET, it should be MSHR_SET (which may be different than the number of sets in the cache HPDCACHE_SETS). Another thing, is that you still need to repeat the MSHR_SET in order to exploit all the MSHR as the MSHR has MSHR_WAYS per MSHR_SET. Thus, I do not know how you generate the addresses, maybe it is already as I propose hereafter:

req.address = (random() / CL_BYTES) * CL_BYTES            // align the base address to a cacheline
while (true) {
    req.address = req.address + CL_BYTES
}

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 340: Read only with large memory back pressure

Test Case: Read only with large memory back pressure

Verification Goal: Purpose: The aim of this performance test is to show the benefits of having a MSHR on the performance. 
Check the bandwidth of cached reads at the core interface of HPDcache when a read arrives with no inter request delay. 
If memory response reply with a delay of 2*MSHR_SETS*MSHR_WAYS cycle(more realistic delay), the cache should be able to accept a read miss every third cycle. 
Tarrget:    Load Performance maximising the use of MSHR 
Scenario: Load only test
Stimulie: Test uses random access sequence. 
Requirements: 
Following condition are respected to get full performance 
1. Load accesses are cacheable 
2. There is no conflit within the MSHR (An access should not be made on a SET which already has an ongoing miss) 
3. There is no inter request delay @core interface 
4. Memory response model reply within minimum delay possible 
Constraints: 
The wbuf and rtab should be configurered to get the maximum outuput. Following values are used:
                 m_cfg_rtab_single_entry = 0

Criteria Pass Fail: Criteria (PASS/FAIL): 
A check is performed at the end of test in report phase. 
The cache is expected to accept  NB_TXN loads in 2*3*NB_TXN cycles.
I would generate the address as above (cf 2.5.2)

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 341: Write(30%) Read(70% ) with no memory back pressure

Test Case: Write(30%) Read(70% ) with no memory back pressure

Verification Goal: Purpose : Check the bandwidth of cached reads (70%) and cached writes(30%) at the core interface of HPDcache when reads and writes arrive with no inter request delay. 
Target     : Read and Write Performance 
Scenario:  Test is constraint to have 70% cacheable read and 30% cacheable  write. 
Stimuli   :  Test is derived from random access tests.  
Requirements: 
Following condition are respected to get full performance 
1. Load/Store accesses are cacheable 
2. There is no conflit within the MSHR (An access should not be made on a SET which already has an ongoing miss) 
3. There is no inter request delay @core interface 
4. Memory response model reply within minimum delay possible 
Constraints: 
OP = HPDCACHE_REQ_LOAD(70%), HPDCACHE_REQ_STORE(30%). Uncacheable = 0. 
5. The wbuf and rtab should be configurered to get the maximum outuput. Following values are used:
                 m_cfg_wbuf_threshold = > 2
                 m_cfg_wbuf_reset_timecnt_on_write = 1;
                 m_cfg_wbuf_sequential_waw = 0;
                 m_cfg_wbuf_inhibit_write_coalescing = 0;
                 m_cfg_rtab_single_entry = 0

Criteria Pass Fail: A check is performed at the end of test in report phase. 
The cache is expected to accept  NB_TXN loads/stores in 3*NB_TXN cycles.

Test Type: Constrained-Random

Coverage Method: Testcase

## Item 342 - Requirement 28.0: Event write req

Requirement ID: 28.0

Feature: The cache provides a set of pulse event counters which output one-cycle pulses each time a given event occurs

Sub Feature: evt_o.write_req --> pulses on a write request accepted

Feature Description: The cache provides a set of pulse event counters which output one-cycle pulses each time a given event occurs

Test Case: Event write req

Verification Goal: Purpose: Coverage that all values of this signal have been covered
Method: Code coverage toggle 
Buckets: 1 bit => 2 buckets

Coverage Method: Code Coverage

## Item 343: Event read req

Sub Feature: evt_o.read_req --> pulses on a read request accepted

Test Case: Event read req

Verification Goal: Purpose: Coverage that all values of this signal have been covered
Method: Code coverage toggle 
Buckets: 1 bit => 2 buckets

Coverage Method: Code Coverage

## Item 344: Evetn prefetch req

Sub Feature: evt_o.prefetch_req --> pulses on a  prefetch request accepted

Test Case: Evetn prefetch req

Verification Goal: Purpose: Coverage that all values of this signal have been covered
Method: Code coverage toggle 
Buckets: 1 bit => 2 buckets

Coverage Method: Code Coverage

## Item 345: Event uncached req

Sub Feature: evt_o.uncached_req --> pulses on a  uncached request accepted

Test Case: Event uncached req

Verification Goal: Purpose: Coverage that all values of this signal have been covered
Method: Code coverage toggle 
Buckets: 1 bit => 2 buckets

Coverage Method: Code Coverage

## Item 346: Event cmo req

Sub Feature: evt_o.cmo_req --> pulses on a  CMO request accepted

Test Case: Event cmo req

Verification Goal: Purpose: Coverage that all values of this signal have been covered
Method: Code coverage toggle 
Buckets: 1 bit => 2 buckets

Coverage Method: Code Coverage

## Item 347: Event accepted req

Sub Feature: evt_o.accepted_req --> pulses on a request accepted (any type)

Test Case: Event accepted req

Verification Goal: Purpose: Coverage that all values of this signal have been covered
Method: Code coverage toggle 
Buckets: 1 bit => 2 buckets

Coverage Method: Code Coverage

## Item 348: Event cache write miss

Sub Feature: evt_o.cache_write_miss --> pulses on a  Write miss event

Test Case: Event cache write miss

Verification Goal: Purpose: Coverage that all values of this signal have been covered
Method: Code coverage toggle 
Buckets: 1 bit => 2 buckets

Coverage Method: Code Coverage

## Item 349: Event cache read miss

Sub Feature: evt_o.cache_read_miss --> pulses on a  Read miss event

Test Case: Event cache read miss

Verification Goal: Purpose: Coverage that all values of this signal have been covered
Method: Code coverage toggle 
Buckets: 1 bit => 2 buckets

Coverage Method: Code Coverage

## Item 350: Event req on hold

Sub Feature: evt_o.req_onhold --> pulses on a  Request put on-hold in the RTAB

Test Case: Event req on hold

Verification Goal: Purpose: Coverage that all values of this signal have been covered
Method: Code coverage toggle 
Buckets: 1 bit => 2 buckets

Coverage Method: Code Coverage

## Item 351 - Requirement 28.0: Event req on hold mshr

Requirement ID: 28.0

Feature: The cache provides a set of pulse event counters which output one-cycle pulses each time a given event occurs

Sub Feature: evt_o.write_req --> pulses on a write request accepted

Feature Description: The cache provides a set of pulse event counters which output one-cycle pulses each time a given event occurs

Test Case: Event req on hold mshr

Verification Goal: Purpose: Coverage that all values of this signal have been covered
Method: Code coverage toggle 
Buckets: 1 bit => 2 buckets

Coverage Method: Code Coverage

## Item 352: Event req on hold wbuf

Sub Feature: evt_o.req_onhold_wbuf --> pulses on a  Request put on-hold because of a WBUF conflict

Test Case: Event req on hold wbuf

Verification Goal: Purpose: Coverage that all values of this signal have been covered
Method: Code coverage toggle 
Buckets: 1 bit => 2 buckets

Coverage Method: Code Coverage

## Item 353: Event req on hold rollback

Sub Feature: evt_o.req_onhold_rollback --> pulses on a Request put on-hold (again) after a rollback

Test Case: Event req on hold rollback

Verification Goal: Purpose: Coverage that all values of this signal have been covered
Method: Code coverage toggle 
Buckets: 1 bit => 2 buckets

Coverage Method: Code Coverage

## Item 354: Event req stall

Sub Feature: evt_o.stall --> pulses on a  Cache stalls request event

Test Case: Event req stall

Verification Goal: Purpose: Coverage that all values of this signal have been covered
Method: Code coverage toggle 
Buckets: 1 bit => 2 buckets

Coverage Method: Code Coverage

## Item 355 - Requirement 29.0: phys_indexed

Requirement ID: 29.0

Feature: Support Physical or Virtual Indexing

Sub Feature: It allows the address and physical memory attributes (PMA) to be sent by the requesters in two different
(but consecutive) cycles. 
The address is split in 2 different parts: one for offset and other for physical tag
This kind of indexing is named Virtually-Indexed Physically-Tagged (VIPT).

Feature Description: Support Physical or Virtual Indexing

Test Case: phys_indexed

Verification Goal: Purpose: Coverage of each value of this signal
Sampling Event: The coverage is sampled when a valid request is received. 
Buckets: 3

Coverage Method: Functional Coverage

## Item 356: Abort Reponse Check

Sub Feature: When using the virtual indexing, the requester can abort the request during the second cycle of the addressing pipeline. 
In that case, the requester needs to set the req_abort signal to 1. And cache responds with a signal is_aborted (if need_rsp = 1)

Test Case: Abort Reponse Check

Verification Goal: Purpose: The abort responses are generated correctly. 
Description: If a request is generated with abort field set, in the case of virtually indexed addressing, the cache respond with abort flag. Check abort response in following cases.
This check if performed when the cache response is received

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 357: abort coverage

Test Case: abort coverage

Verification Goal: Purpose: Coverage of each value of this signal
Sampling Event: The coverage is sampled when a valid request is received. 
Buckets: 2

Coverage Method: Functional Coverage

## Item 358: Store with Abort response

Test Case: Store with Abort response

Verification Goal: Purpose: Cover that a Store with an abort is followed by a LOAD/AMOS (no rsp )on the same line. Cover for cache and uncached transactions. 
Trigger and Clocking Events: The clock is a core clk. The assertion is triggered by when a valid store is recieved.

Coverage Method: Functional Coverage

## Item 359: Load with Abort response

Test Case: Load with Abort response

Verification Goal: Purpose: Cover that a Load with an abort response (memory)  is followed by a LOAD/AMO (no rsp ) on the same line. Cover for cache and uncached transactions. 
Trigger and Clocking Events: The clock is a core clk. The assertion is triggered by when a valid load is recieved.

Coverage Method: Functional Coverage

## Item 360: AMOs with Abort  response

Test Case: AMOs with Abort  response

Verification Goal: Purpose: Cover that an AMO with an abort  is followd by a LOAD/AMO(no rsp) on the same line
Trigger and Clocking Events: The clock is a core clk. The assertion is triggered by when a valid AMO is recieved.

Coverage Method: Functional Coverage

## Item 361 - Requirement 30.0: test_hpdcache_multiple_directed_addr_bPLRU_prediction

Requirement ID: 30.0

Feature: HPDcache implements by default a PLRU replacement policy

Sub Feature: The HPDcache must use this policy to select the victim way where a new cacheline is written

Feature Description: HPDcache implements by default a PLRU replacement policy

Test Case: test_hpdcache_multiple_directed_addr_bPLRU_prediction

Verification Goal: Purpose:  The purpose of this the test is to check the PLRU eviction algorithm using a black box approach.
Target:  PLRU algorithm 
Scenario:  To avoid temporal conflicts between request and refill response which cannot be predicted, in this test each request is sent every N( = 15) cycles and the memory response model is configured to respond with zero delay. We require that the response from the previous request is received before the next request is issued. Otherwise it is impossible to predict the PLRU behavior using a black box approach.
Stimuli :  The sequence is derived from 1.2.1
Requirement :  This test  respects the constraints related to hpdcache protocol. This test enables a UVM based model in the SB which  does the black box prediction of the PLRU under a limited set of stimulus which is respected by this test.
Constraint       :  Every field of the transaction is randomized except Set, address, uncacheable (always 0 -> cacheable) and inter request delay. Memory response model is configured to respond with zero delay. 
Note:  We note that the environment also has a SV based PLRU model which is bind directly to the memory control module to do the exact prediction of the PLRU using the signals from memory control module (white box assertion based checking).   In the UVM based model, the PLRU prediction takes into accoun the type of transactions.

Criteria Pass Fail: The TB has UVM based PLRU model which flags a UVM_ERROR in case PLRU algorithm is not respected. A tag directory is maintained which helps predict hit and miss in this particular scenario.

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 362: test_hpdcache_multiple_random_requests_in_region_bPLRU_prediction

Test Case: test_hpdcache_multiple_random_requests_in_region_bPLRU_prediction

Verification Goal: This is the variant of test 1.2.2 where this test is derived from the test 1.1.4.
Scenario: The idea is to perform black box PLRU prediction when the memory accesses are distributed across a memory region. In 1.2.2 the accesses where focused on a single set to provoke frequent evictions.

Criteria Pass Fail: TB has multiple checks, in case of an error as UVM_ERROR is flagged. Please refer to section "Environnement Checks( section: 6)".

Test Type: Directed Self-Checking

Coverage Method: Testcase

## Item 363: PLU Check

Test Case: PLU Check

Verification Goal: Purpose: Check if PLRU algorithm is correct. 
Method: A SV based PLRU model is bind to the mem_ctrl instance in the HPDcache to be able the check the correct functioning of PLRU.

Test Type: ENV capability, not specific test

Coverage Method: Functional Coverage

## Item 364: Bit-PLRU configuration variable

Test Case: Bit-PLRU configuration variable

Verification Goal: Purpose:  To make sure that plru bit is enabled. Cover that in the TB the plru is enabled (in the directed test) 
Buckets: 1 
Sampling Event: Sample when PLRU check is done.

Coverage Method: Functional Coverage

## Item 365: Least Recently used bit

Sub Feature: Replacement policy must provide one bit per cache line

Test Case: Least Recently used bit

Verification Goal: Purpose: To make sure every bit of PLRU has been covered. Cover that each PLRU bit is least recently used (first 0) -> for a PLRU status of 4 bits 
XX10
X101
1011 
0111
Sampling Event: When the PLRU bit is flipped.

Coverage Method: Functional Coverage

## Item 366: Each operation with PLRU bit set

Test Case: Each operation with PLRU bit set

Verification Goal: Purpose: Every cache operation on each line with PLRU bit set
Sampling Event: When a write/read hit is predicted by the model of PLRU (SV base model)

Coverage Method: Functional Coverage

## Item 367: Least Recently used bit

Sub Feature: LRU bits are 0 at reset and mut be udated at each read, store, atomic operation.

Test Case: Least Recently used bit

Verification Goal: Purpose: To make sure every bit of PLRU has been covered. Cover that each PLRU bit is least recently used (first 0) -> for a PLRU status of 4 bits 
XX10
X101
1011 
0111
Sampling Event: When the PLRU bit is flipped.

Coverage Method: Functional Coverage

## Item 368: HIT (write/read) each line

Test Case: HIT (write/read) each line

Verification Goal: Purpose: Cover that each line is a read/write hit 
Sampling Event: When a write/read hit is predicted by the model of PLRU (SV base model)

Coverage Method: Functional Coverage

## Item 369: MISS(write/read) each line

Sub Feature: LRU bits are updated by refill

Test Case: MISS(write/read) each line

Verification Goal: Purpose: Cover that each line is a read/write miss
Sampling Event: When a write/read hit is predicted by the model of PLRU (SV base model)

Coverage Method: Functional Coverage

## Item 370: Evict each line

Test Case: Evict each line

Verification Goal: Purpose:cover that each line has been evicted 
Sampling Event: When a write/read hit is predicted by the model of PLRU (SV base model)

Coverage Method: Functional Coverage

## Item 371: MISS(write/read) each line

Sub Feature: Cache controler applies the plru algomrithm in the case of a hit for a request from a requester

Test Case: MISS(write/read) each line

Verification Goal: Purpose: Cover that each line is a read/write miss
Sampling Event: When a write/read hit is predicted by the model of PLRU (SV base model)

Coverage Method: Functional Coverage

## Item 372: Evict each line

Sub Feature: Cache controler applies the plru algorithm in the case of a refill, once victim way is selected

Test Case: Evict each line

Verification Goal: Purpose:cover that each line has been evicted 
Sampling Event: When a write/read hit is predicted by the model of PLRU (SV base model)

Coverage Method: Functional Coverage

## Item 373: Store with Error response

Sub Feature: In the case of error responses, PLRU is not updated

Test Case: Store with Error response

Verification Goal: Purpose: Cache is updated in the case of cached store in any case error or not.  Cover that a Store with an error response(memory ) is followed by a LOAD/AMOS (no rsp )on the same line. Cover for cache and uncached transactions.  
Trigger and Clocking Events: The clock is a core clk. The assertion is triggered by when a valid store is recieved.

Coverage Method: Functional Coverage

## Item 374: Load with Error response

Test Case: Load with Error response

Verification Goal: Purpose: Cover that a Load (miss) with an error response (memory)  is followed by a LOAD/AMO (no rsp ) on the same line. Cover for cache and uncached 
transactions.
Trigger and Clocking Events: The clock is a core clk. The assertion is triggered by when a valid load is recieved.

Coverage Method: Functional Coverage

## Item 375: AMOs with Error response

Test Case: AMOs with Error response

Verification Goal: Purpose: Cover that an AMO with an error response(memory) is followd by a LOAD/AMO(no rsp) on the same line
Trigger and Clocking Events: The clock is a core clk. The assertion is triggered by when a valid AMO is recieved.

Coverage Method: Functional Coverage

## Item 376: Each CMOs with PLRU bit set

Sub Feature: In the case of CMOs (fence or invalid), PLRU is not updated

Test Case: Each CMOs with PLRU bit set

Verification Goal: Purpose: CMOs operation does not update the PLRU. In this coverage it is verified that a CMO operation is performed in each PLRU bit set 
Sampling Event: When a write/read hit is predicted by the model of PLRU (SV base model)

Coverage Method: Functional Coverage

## Item 377 - Requirement 31.0: No signals with X/Z values when valid

Requirement ID: 31.0

Feature: HPDCache must make sure no X is propgated

Sub Feature: HPDcache must make sure that when ready & valid = 1,  no signal is at X or Z on the respective interface

Feature Description: HPDCache must make sure no X is propgated

Test Case: No signals with X/Z values when valid

Verification Goal: Purpose: Assertion to check that no signal got an X/Z values in it when the valid signal is enabled.
Following are the signals. 
Core request 
tag
offset
wdata
op
be
size
uncacheable
sid
tid
need_rsp
phy_indexed 
pma.io
pma.uncached 

Core response 
rdata
sid
tid
error
abort 
Method: SV Assertion $unknown is used to check the signals. 
Link: hpdcache_sva.sv

Coverage Method: Assertion Coverage

## Item 378: No signals with X/Z values when valid

Test Case: No signals with X/Z values when valid

Verification Goal: Purpose: Assertion to check that no signal got an X/Z values in it when the valid signal is enabled
memory request interface (read and write) 
mem_req_addr
mem_req_len
mem_req_size
mem_req_id
mem_req_command
mem_req_atomic
mem_req_cacheable

memory write data request 
mem_req_w_data
mem_req_w_be
mem_req_w_last

memory read respons interface (read and write) 
mem_resp_r_error
mem_resp_r_id
mem_resp_r_data
mem_resp_r_last

memory write respons interface (read and write) 
mem_resp_w_is_atomic
mem_resp_w_error
mem_resp_w_id
Method: SV Assertion $unknown is used to check the signals. 
Link: hpdcache_sva.sv

Coverage Method: Assertion Coverage